FINAL REGISTRATION REPORT Part B Section 5 Analytical Methods Detailed summary of the risk assessment

Product code: CHR/H/IZOXACYP 250 SC Product name(s): Metida Plus 250 SC Taizza Plus 250 SC Chemical active substance: Isoxaflutole, 250 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: Innvigo Sp. z o. o. Submission date: April 2022 Finalisation date: December 2022; August 2023

Version history

| When | What |
| --- | --- |
| 12/2022 | ZRMs Assessment |
| August 2023 | Final version of RR after commenting period. |

Table of Contents

5	Analytical methods	4

5.1	Conclusion and summary of assessment	4

5.2	Methods used for the generation of pre-authorization data (KCP 5.1)	4

5.2.1	Analysis of the plant protection product (KCP 5.1.1)	4

5.2.1.1	Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)	4

5.2.1.2	Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)	7

5.2.1.3	Description of analytical methods for the determination of formulants (KCP 5.1.1)	8

5.2.1.4	Applicability of existing CIPAC methods  (KCP 5.1.1)	11

5.2.2	Methods for the determination of residues (KCP 5.1.2)	11

5.3	Methods for post-authorization control and monitoring purposes (KCP 5.2)	12

5.3.1	Analysis of the plant protection product (KCP 5.2)	13

5.3.2	Description of analytical methods for the determination of residues isoxaflutole (KCP 5.2)	13

5.3.2.1	Overview of residue definitions and levels for which compliance is required	13

5.3.2.2	Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)	14

5.3.2.3	Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)	14

5.3.2.4	Description of methods for the analysis of soil (KCP 5.2)	14

5.3.2.5	Description of methods for the analysis of water (KCP 5.2)	15

5.3.2.6	Description of methods for the analysis of air (KCP 5.2)	15

5.3.2.7	Description of methods for the analysis of body fluids and tissues (KCP 5.2)	16

Appendix 1	Lists of data considered in support of the evaluation	17

Appendix 2	Detailed evaluation of submitted analytical methods	22

A 2.1	Analytical methods for isoxaflutole	22

A 2.1.1	Methods used for the generation of pre-authorization data (KCP 5.1)	22

A 2.1.2	Methods for post-authorization control and monitoring purposes (KCP 5.2)	22

# Analytical methods

Data matching studies for isoxaflutole have been evaluated by RMS and as a result of the assessment all reports were accepted and considered as equivalent to protected studies. Therefore, to support the authorization of CHR/H/IZOXACYP 250 SC (Taizza Plus 250 SC, Metida Plus 250 SC) INNVIGO is allowed to refer to EU approved reports.

## Conclusion and summary of assessment

Sufficiently sensitive and selective analytical methods are available for the active substance(s) and relevant impurities in the plant protection product.

Noticed data gaps:

- none

Sufficiently sensitive and selective analytical methods are available for all analytes included in the residue definitions.

Noticed data gaps are:

- ILV for drinking water is required according to Regulation (EU) No 284/2013 (post-registration requirement)

- analytical methods for body fluids and tissues are required according to Regulation (EU) No 284/2013 (post-registration requirement)

| Commodity/crop | Supported/ Not supported |
| --- | --- |
| Maize | Supported |

ILV for drinking water is required according to Regulation (EU) No 284/2013

## Methods used for the generation of pre-authorization data (KCP 5.1)

### Analysis of the plant protection product (KCP 5.1.1)

#### Determination of active substance and/or variant in the plant protection product (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of active substance, isoxaflutole, in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical method is suitable for the determination of active substance isoxaflutole in plant protection product Taizza Plus 250 SC. The proposed analytical method has been fully validated in terms of specificity, linearity, repeatability, and accuracy. Proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | KCP 5.1.1/01 |
| --- | --- |
| Report | Patrzałek M., Validation of analytical method for Izoxaflutole 250 SC (CHR/H/IZOXACYP 250 SC) for determination of isoxaflutole, 2021, Study code: ICB/10/2021 |
| Guideline(s): | Conducting of analytical method validation – own Standard Operational Procedure SPB/179. |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Validation of method for determination of the content of isoxaflutole

Validation was carried out according to Standard Operational Procedure SPB/179. Content of isoxaflutole at a level of 250 g/L in the test item was determined accordingly by liquid chromatography with diode array detection (HPLC-DAD).

Equipment and materials

- acetonitrile HPLC (VWR),

- 85% phosphoric acid (Sigma Aldrich),

- water ultra-purification unit HLP 5UV,

- isoxaflutole standard; Sigma Aldrich; batch BCBT2782,

- isoxaflutole standard solution in acetonitrile,

- working standard solution of isoxaflutole in acetonitrile,

- analytical balance – accuracy 0.0001 g (Ohaus, Switzerland),

- liquid chromatography with diode array detection (Shimadzu, Japan),

- chromatography column type C18, 150 mm x 4.6 mm; 5 µm (Kinetex, Phenomenex),

- chromatography column type C18, 250 mm x 4.6 mm; 5 µm (Luna®, Phenomenex),

- chromatographic vials 1.5 ml with septa buthyl/Teflon, - volumetric flask A class 10 mL,

- measuring syringes 10 µL, 50 µL 250 µL, 500 µL, 1000 µL.

Concentrations of the test item

Concentration taking into account the purity of the standard [mg/mL]: 1.02691

Concentrations of working solutions [µg/mL]:

            - 1.027

            - 5.135

            - 10.269

            - 20.538

            - 41.076

Chromatography parameters.

Chromatography parameters were determined during preliminary assessment.

Primary chromatographic system:

Column: C18, 150 mm x 4.6 mm; 5µm (Penomenex, Kinetex)

Analysis time: 15 min

Flow: 1.4 mL/min

Column temperature: 25 °C

Mobile phase: A – Acetonitrile, B – 0.1% H3PO4

Isocratic program: phase A – 45%, phase B – 55%

Injection: 20 µL

Detector DAD: 269 nm

Secondary chromatographic system:

Column: C18, 250 mm x 4.6 mm; 5µm (Penomenex, Luna (2))

Analysis time: 30 min

Flow: 1.4 mL/min

Column temperature: 25°C

Mobile phase: A – Acetonitrile, B – 0.1% H3PO4

Isocratic program: phase A – 45%, phase B – 55%

Injection: 20 µL

Detector DAD: 269 nm

Validation - Results and discussions

Table 5.2-1:	Methods suitable for the determination of active substance isoxaflutole in plant protection product CHR/H/IZOXACYP 250 SC

|  | Isoxaflutole |
| --- | --- |
| Author(s), year | Partzałek M., 2021 |
| Principle of method | HPLC-DAD |
| Linearity (n=5) (linear between 1.027 – 41.076 mg/L / range of the declared content) (correlation coefficient, expressed as r) | Primary chromatographic system: Equation of calibration curve: f(x)=3.77086×10-5 x + 0.1116111 R = 0.9999831 R2 = 0.9999661 (range of concentrations: 1.027 – 303.886 41.076 µg/mL) Secondary chromatographic system: Equation of calibration curve: f(x)=3.86531×10-5 x + 0.0759925 R = 0.9999830 R2 = 0.9999660 (range of concentrations: 1.027 – 303.886 41.076 µg/mL) |
| Precision – Repeatability Mean N = 5 (%RSD) | Primary chromatographic system: 100% without standard addition: %RSD=0.26%; RSDr=1.69%; Horrat value=0.15 LOQ level: %RSD=1.14%; RSDr=2.68%; Horrat value=0.43 uLOQ level: %RSD=1.28%; RSDr=1.61%; Horrat value=0.80 Secondary chromatographic system: 100% without standard addition: %RSD=0.49%; RSDr=1.69%; Horrat value=0.29 LOQ level: %RSD=2.35%; RSDr=2.68%; Horrat value=0.88 uLOQ level: %RSD=1.41%; RSDr=1.61%; Horrat value=0.88 |
| Accuracy n = 5 (% Recovery) | Primary chromatographic system: 100% without standard addition Marginal recovery: 98.43 – 102.01% (within the limits: 97-103%) LOQ level: 105.9% uLOQ level: 101.9% The results fulfils the acceptance criterion (90 – 110%). Secondary chromatographic system: 100% without standard addition Marginal recovery: 98.36 – 101.31% (within the limits: 97-103%) LOQ level: 100.1% uLOQ level: 100.9% The results fulfils the acceptance criterion (90 – 110%). |
| Interference/ Specificity | Specificity of the method was evaluated based on the analysis of chromatograms for blank samples (placebo) against samples of placebo spiked with isoxaflutole standard. Analysis showed no overlapping of determined substance signal with the signals of matrix components under method conditions hence method specificity criterion is fulfilled. |
| LOQ | The limit of quantification (LOQ) was defined as the lowest concentration of standard - 1.027 µg/mL, which was determined with an acceptable recovery. |
| Comment | - |

Conclusion

The method for determination of active substance isoxaflutole in IZOXAFLUTOLE 250 SC (CHR/H/IZOXACYP) preparation is specific. The validation parameters for linearity, instrument precision, repeatability and accuracy are within the acceptance range. The determined average content of isoxaflutole in IZOXAFLUTOLE 250 SC (CHR/H/IZOXACYP) is: 21.65 ± 0.08%.

#### Description of analytical methods for the determination of relevant impurities (KCP 5.1.1)

Not relevant.

#### Description of analytical methods for the determination of formulants (KCP 5.1.1)

An overview on the acceptable methods and possible data gaps for analysis of formulant, cyprosulfamide, in plant protection product is provided as follows:

| Comments of zRMS: | The proposed analytical method is suitable for the determination of formulant cyprosulfamide in plant protection product Taizza Plus 250 SC. The proposed analytical method has been fully validated in terms of specificity, linearity, repeatability, and accuracy. Proposed method fulfils the requirements of SANCO/3030/99 rev.5 guidance. The validation of the analytical method has been accepted. |
| --- | --- |

| Reference: | KCP 5.1.1/02 |
| --- | --- |
| Report | Łebek B., Validation of analytical method for Izoxaflutole 250 SC (CHR/H/IZOXACYP 250 SC) for determination of cyprosulfamide, 2022, Study code: ICB/6/2022 |
| Guideline(s): | Conducting of analytical method validation – own Standard Operational Procedure SPB/179. |
| Deviations: | No |
| GLP: | Yes |
| Acceptability: | Yes |

Materials and methods

Validation of methods for determination of the chemical content of cyprosulfamide

Validation was carried out according to Standard Operational Procedure SPB/179. Content of cyprosulfamide at a level of 250 g/L in the test item was accordingly determined by liquid chromatography with diode array detection (HPLC-DAD).

Equipment and materials

- acetonitrile HPLC (VWR),

- 85% phosphoric acid (Merck),

- placebo,

- cyprosulfamide standard; Sigma Aldrich; batch BCCF9275,

- standard stock solution of cyprosulfamide in acetonitrile, for calibration (Table 2),

- working standard solutions of cyprosulfamide in acetonitrile, for calibration (Table 3),

- standard stock solution of cyprosulfamide in acetonitrile, for determination of recovery (Table 4),

- analytical balance – accuracy 0.0001 g, WP/16 (Ohaus, Switzerland),

- liquid chromatograph with diode array detection, WP/19 (Shimadzu, Japan),

- chromatography column type C18, 150 mm x 4.6 mm; 5 µm (Kinetex, Phenomenex), K/12/HPLC,

- liquid chromatograph with diode array detection, WP/42 (Shimadzu, Japan),

- chromatography column type C18(2), 250 mm x 4.6 mm; 5 µm (Luna, Phenomenex), K/17/HPLC,

- chromatographic vials 1.5 ml with septa buthyl/Teflon,

- volumetric flasks A class 10 mL,

- measuring syringes 10 µL, 100 µL, 250 µL, 500 µL, 1000 µL.

Concentrations of the test item

Concentration taking into account the purity of the standard [mg/mL]: 1.10778

Concentrations of working solutions [µg/mL]:

            - 1.108

            - 5.539

            - 11.078

            - 22.156

            - 44.311

Chromatography parameters.

Chromatography parameters were determined during preliminary assessment.

Primary chromatographic system:

Column: K/12/HPLC

Analysis time: 15 min

Flow: 1.4 mL/min

Column temperature: 25 °C

Mobile phase: A – Acetonitrile, B – 0.1% H3PO4

Isocratic program: phase A – 45%, phase B – 55%

Injection: 20 µL

Detector DAD: 269 nm

Secondary chromatographic system:

Column: K/17/HPLC

Analysis time: 30 min

Flow: 1.4 mL/min

Column temperature: 25°C

Mobile phase: A – Acetonitrile, B – 0.1% H3PO4

Isocratic program: phase A – 45%, phase B – 55%

Injection: 20 µL

Detector DAD: 269 nm

Validation - Results and discussions

Table 5.2-2:	Methods suitable for the determination of cyprosulfamide in plant protection product CHR/H/IZOXACYP 250 SC

|  | Cyprosulfamide |
| --- | --- |
| Author(s), year | Łebek B., 2022 |
| Principle of method | HPLC-DAD |
| Linearity N=5 (linear between 1.108 – 44.311 mg/L / % range of the declared content) (correlation coefficient, expressed as r) | Primary chromatographic system: Equation of calibration curve: f(x) = 6.08568×10-5 x - 0.104850 R = 0.9999272 R2 = 0.9998544 (range of concentrations: 1.108 – 44.311 µg/mL) Secondary chromatographic system: Equation of calibration curve: f(x) = 6.31410×10-5 x - 0.0781263 R = 0.9999120 R2 = 0.9998240 (range of concentrations: 1.108 – 44.311 µg/mL) |
| Precision – Repeatability Mean N = 5 (%RSD) | Primary chromatographic system: %RSD=0.13%; RSDr=1.68%; Horrat value = 0.08 LOQ level: %RSD=0.57%; RSDr=2.68%; Horrat value=0.21 uLOQ level: %RSD=0.81%; RSDr=1.61%; Horrat value=0.5 Secondary chromatographic system: %RSD=0.24%; RSDr=1.68%; Horrat value = 0.14 LOQ level: %RSD=1.42%; RSDr=2.68%; Horrat value=0.53 uLOQ level: %RSD=0.57%; RSDr=1.61%; Horrat value=0.35 |
| Accuracy n = 5 (% Recovery) | Primary chromatographic system Marginal recovery: 98.88 - 100.78% (within the limits: 97-103%) LOQ level: 92.9% (average) uLOQ level: 100.8% (average) The results fulfils the acceptance criterion (90 – 110%). Secondary chromatographic system Marginal recovery: 98.30 – 100.27% (within the limits: 97-103%) LOQ level: 92.5% (average) uLOQ level: 100.1% (average) The results fulfils the acceptance criterion (90 – 110%). |
| Interference/ Specificity | Specificity of the method was evaluated based on the analysis of chromatograms for placebo and samples against chromatograms of standard cyprosulfamide and peak purity. Analysis showed no overlapping of determined ingredient signal with the signals of matrix components under method conditions, hence method specificity criterion is fulfilled. |
| LOQ | The limit of quantification (LOQ) was defined as the lowest concentration of standard – 1.108 µg/mL, which was determined with an acceptable recovery. |
| Comment | - |

Conclusion

The method for determination of formulant cyprosulfamide in IZOXAFLUTOLE 250 SC (CHR/H/IZOXACYP) preparation is specific. The validation parameters for linearity, instrument precision, repeatability and accuracy are within the acceptance range. The determined average content of cyprosulfamide in IZOXAFLUTOLE 250 SC (CHR/H/IZOXACYP) is: 22.12±0.04%.

#### Applicability of existing CIPAC methods  (KCP 5.1.1)

Not relevant.

### Methods for the determination of residues (KCP 5.1.2)

An overview on the acceptable methods and possible data gaps for analysis of residues of isoxaflutole for the generation of pre-authorization data is given in the following table. For the detailed evaluation of new studies it is referred to Appendix 2.

Table 5.2-2:	Validated methods for the generation of pre-authorization data

| Component of residue definition: isoxaflutole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| Food/feed of plant origin (Residues) | Primary | 0.01 mg/kg | GC-MS | Manley, 1995 ref 4.2.1/0.1 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
|  | Confirmatory (if required) | Not required |  |  |
| Animal products, food of animal origin (Residues) | Primary | 0.01-0.02 mg/kg | HPLC-UV | Guillet 1995b ref. 4.2.5/01 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
|  | Confirmatory (if required) | 0.01-0.02 mg/kg | HPLC-UV | Hunt, T.W. and Lopes, A., 1995 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
| Soil (Environmental fate) | Primary | LOQ =0.005 mg/kg | HPLC-UV | Guillet, M., Venet, C. & Simonin, B., 1995 ref 4.2.2/01 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
|  | Confirmatory (if required) | LOQ =0.005 mg/kg | HPLC-UV | Schuster, L.L., 1995, ref 4.2.2/02 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
| Water (surface, ground and drinking water ) (Environmental fate) | Primary | LOQ = 0.002 mg/l. | HPLC-UV | Le Gren 1995a ref. 4.2.3/01 DAR Isoxaflutole (1997) – Vol-ume 3 Annex B.4 – Analytical methods |
|  | Confirmatory (if required) | LOQ = 0.001 mg/l. | HPLC-UV | Le Gren 1995b ref. 4.2.3/02 DAR Isoxaflutole (1997) – Vol-ume 3 Annex B.4 – Analytical methods |
| Air (Environmental fate) | Primary | 0.002 μg/m3 | HPLC-UV | Corgier, M. and Turier, G.,1995 DAR Isoxaflutole (1997) – Vol-ume 3 Annex B.4 – Analytical methods |
|  | Confirmatory (if required) | Not required |  |  |
| Soil, water (Efficacy) | Primary | Not required |  |  |
|  | Confirmatory (if required) |  |  |  |
| Feed, body fluids, (Toxicology) | Primary | No methods were supplied for the determination of isoxaflutole or metabolites in these matrices. A case has been made for not providing these as the active substance is not classified as toxic or very toxic which is acceptable. |  |  |
|  | Confirmatory (if required) |  |  |  |
| Body fluids, air, (Exposure) | Primary | Not available |  |  |
|  | Confirmatory (if required) |  |  |  |
| Soil, water. (Ecotoxicology) | Primary | Methods in soil, water, sediment, feed and any additional matrices used in support of ecotoxicology studies. No general analytical methods were developed for risk assessment apart those reported as specific in studies on route and rate. For details please refer to section B8 Isoxaflutole DAR 21997 and Addendum 2002. |  |  |
|  | Confirmatory (if required) |  |  |  |

## Methods for post-authorization control and monitoring purposes (KCP 5.2)

Data provided on Annex I inclusion is sufficient for post-authorizations methods. No new methods are necessary since all data is described and presented in Table 5.2-3 in point KCP 5.1.2.

Therefore, the new studies for CHR/H/IZOXACYP 250 SC have not been performed.

### Analysis of the plant protection product (KCP 5.2)

For active substance isoxaflutole all presented methods are sufficient and no new methods are necessary. Please refer to KCP 5.2

### Description of analytical methods for the determination of residues isoxaflutole (KCP 5.2)

#### Overview of residue definitions and levels for which compliance is required

Compared to the residue definition proposed in the Draft Assessment Report (incl. its addenda) the current legal residue definition is identical.

Table 5.3-3:	Relevant residue definitions for monitoring/enforcement and levels for which compliance is required

| Matrix | Residue definition | MRL / limit | Reference for MRL/level Remarks |
| --- | --- | --- | --- |
| Plant, high protein/high starch content (dry commodities) | Isoxaflutole (sum of isoxaflutole and its diketonitrile-metabolite; expressed as isoxaflutole) | LOQ = 0.01 mg/kg Lowest MRL = 0.02 mg/kg | Commission Regulation (EU) 2015/845 of 27 May 2015 |
| Muscle | Not necessary for the representative uses | - | - |
| Milk |  |  |  |
| Eggs |  |  |  |
| Fat |  |  |  |
| Liver, kidney |  |  |  |
| Soil (Ecotoxicology) | Isoxaflutole, RPA 202248, RPA 203328 | 0.005 mg/kg | DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
| Drinking water (Human toxicology) | Sum of isoxaflutole and RPA 202248, RPA 203328 | Limit: 0.1 µg/L | general limit for drinking water |
| Surface water (Ecotoxicology) | Sum of isoxaflutole and RPA 202248, RPA 203328 | 55 µg a.s/L from most sensetive species for Metabolite RPA 202248 Lemna gibba | SANCO/3136/99-Final 7 April 2003 |
| Air | isoxaflutole | LOQ = 2 µg/m3 | AOEL sys/AOEL inhal: 0.02 mg/kg bw/d |
| Tissue (meat or liver) | Sum of isoxaflutole and RPA 202248, RPA 203328 | Not required | Not classified as T / T+ |
| Body fluids |  | Not required | Not classified as T / T+ |

#### Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of isoxaflutole in plant matrices is given in the following tables. For the detailed evaluation of additional studies it is referred to Appendix 2.

Table 5.3-4:	Validated methods for food and feed of plant origin (required for all matrix types, “difficult” matrix only when indicated by intended GAP)

| Component of residue definition: isoxaflutole and its diketonitrile metabolite, expressed as isoxaflutole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing / EU agreed |
| High protein/high starch content (dry) Maize | Primary | 0.01 mg/kg | GC-MS | Manley, 1995 ref 4.2.1/0.1 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
|  | ILV | 0.01 mg/kg | LC-MS/MS | Schuster, L.L., 1995a Manley, 1995 ref 4.2.1/0.1 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
|  | Confirmatory (if required) | Not required |  |  |

Table 5.3-5:	Statement on extraction efficiency

|  | Method for products of plant origin |
| --- | --- |
| Required, available from: | Extraction efficiency was also validated using radio labelled material coming from the metabolism study (Manley, 1995 ref 4.2.1/01). The blank values are generally < 0.001 mg/kg (expressed as isoxaflutole). In some very limited cases, blank values of 0.008 mg/kg have been found. The repeatability of the method has been determined from the recovery efficiency from all the recovery tests (204). The standard deviation was found to be 15 %. |
| Not required, because: | N/A |

#### Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

For all representative uses in maize, there is no requirement for presenting methods for food and feed of animal origin. There is no residue definition for monitoring purposes.

#### Description of methods for the analysis of soil (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of isxoaflutole in soil is given in the following tables. No new methods are necessary.

Table 5.3-6:	Validated methods for soil (if appropriate)

| Component of residue definition: isoxaflutole |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | 0.005 mg/kg | HPLC-UV | Guillet, M., Venet, C. & Simonin, B., 1995 ref 4.2.2/01 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
| Confirmatory | 0.005 mg/kg | HPLC-UV | Schuster, L.L., 1995, ref 4.2.2/02 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |

#### Description of methods for the analysis of water (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of isoxaflutole in surface and drinking water is given in the following tables. No new studies are necessary.

Table 5.3-7:	Validated methods for water (if appropriate)

| Component of residue definition: isoxaflutole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Matrix type | Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Drinking water | Primary | LOQ = 2 μg/L | HPLC-UV | Le Gren, 1995a ref. 4.2.3/01 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
|  | ILV | N/A |  |  |
|  | Confirmatory | LOQ = 2 μg/L | HPLC-UV | Le Gren, 1995b ref. 4.2.3/02 DAR Isoxaflutole (1997) – Volume 3 Annex B.4 – Analytical methods |
| Surface water | Not preformed, not required |  |  |  |

For any special comments or remarkable points concerning the analytical methods for water please refer to Appendix 2.

RMS: An ILV for drinking water is required according to Regulation (EU) No 284/2013.

#### Description of methods for the analysis of air (KCP 5.2)

An overview on the acceptable methods and possible data gaps for analysis of isoxaflutole in air is given in the following tables. No new studies are necessary.

Table 5.3-8:	Validated methods for air (if appropriate)

| Component of residue definition: isoxaflutole |  |  |  |
| --- | --- | --- | --- |
| Method type | Method LOQ | Principle of method (i.e. GC-MS or HPLC-UV) | Author(s), year / missing |
| Primary | LOQ = 2 μg/m3 | LC-UV | DAR Isoxaflutole (1997) – Vol-ume 3 Annex B.4 – Analytical methods |
| Confirmatory | Not required |  |  |

For any special comments or remarkable points concerning the analytical methods for air it is referred to Appendix 2.

#### Description of methods for the analysis of body fluids and tissues (KCP 5.2)

No methods were supplied for the determination of isoxaflutole or metabolites in these matrices. A case has been made for not providing these as the active substance is not classified as toxic or very toxic which is acceptable.

zRMS: nalytical methods for body fluids and tissues are required according to Regulation (EU) No 284/2013 (post-registration requirement)

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 5.1.1/01 | Patrzałek M. | 2021 | Validation of analytical method for Izoxaflutole 250 SC (CHR/H/IZOXACYP 250 SC) for determination of isoxaflutole Company Report No ICB/10/2021 ICB Pharma, 10 Lema Street, 43-600 Jaworzno Poland GLP Unpublished | N | Chemirol |
| KCP 5.1.1/02 | Łebek B. | 2022 | Validation of analytical method for Izoxaflutole 250 SC (CHR/H/IZOXACYP 250 SC) for determination of cyprosulfamide Company Report No ICB/6/2022 ICB Pharma, 10 Lema Street, 43-600 Jaworzno Poland GLP Unpublished | N | Chemirol |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCA 5.1.2/01 | Manley, J. D. | 1995 | Analytical method for the determination of residues of RPA 201772, RPA 202248 and RPA 203328 in maize forage, grain and fodder Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: C021001, Edition Number: M-210419-01-1 Date: 1995-10-24 GLP/GEP: yes, unpublished ...also filed: KCA 4.2 /05 | N | Bayer CropScience |
| KCA 5.1.2/02 | Austin, D .J.; Manley, J. D. | 1995 | Herbicides: RPA201772: The evaluation of alternative procedures to diazomethane for the preparation of esters of RPA203328 for the "Analytical method for the determination of residues of RPA201772, RPA202248 and RPA203328 in maize grain an Rhone-Poulenc Agriculture Ltd., Ongar, Essex, United Kingdom Bayer CropScience, Report No.: R000352, Report includes Trial Nos.: P94/110 Edition Number: M-158450-01-1 Date: 1995-04-28 GLP/GEP: yes, unpublished | N | Bayer CropScience |
| KCA 5.1.2/03 | Schuster, L.L., | 1995a | PR Notice 88-5 Enforcement Method Confirmation for RPA 201772 and its Metabolites RPA 202248 and RPA 203328 in Corn Grain Generated by: ABC Laboratories, Pan-Ag Division, Madera, California, USA Submitted by: Rhône-Poulenc Agro, France Report/file N°: 94454, Docmap n° 44663 Date of report: February 23, 1995 | N | Bayer CropScience |
| KCA 5.1.2/04 | Guillet, M., Venet, C. & Simonin, B., | 1995 | Isoxaflutole and Metabolites: Analytical Method for the Determination of Residues in Soil. Generated by: Rhône-Poulenc Secteur Agro, Lyon, France Submitted by: Rhône-Poulenc Agro, France Report/file N°: AR 106-94 (E), Docmap n° 437490, Date of report: February 2, 1995. | N | Bayer CropScience |
| KCA 5.1.2/05 | Schuster, L.L., | 1995 | Method Confirmation for RPA 201772 and its Metabolites RPA 202248, RPA 203328 and RPA 205834 in Soil. Generated by: ABC Laboratories, Pan-Ag Division, Madera, California, USA Submitted by: Rhône-Poulenc Agro, France Report/file N°: EC-95-305 Date of report: November 7, 1995 | N | Bayer CropScience |
| KCA 5.1.2/06 | Plaisance, R.S., 1995 | 1995 | Validation of Method of Analysis for Isoxaflutole and its Metabolites in/on Agricultural Soil. Generated by: Rhône-Poulenc Ag, Co., USA. Submitted by: Rhône-Poulenc Agro, France. Report/file N°: EC-95-299 Date of report: November 6, 1995 | N | Bayer CropScience |
| KCP 5.1.2/07 | Corgier, M. and Turier, G., | 1995 | Analytical Method for the Determination of Isoxaflutole in Air. Generated by: Rhône-Poulenc Agro, France. Submitted by: Rhône-Poulenc Agro, France. Report/file N°: 94-115 Docmap 438832 Date of report: June 15, 1995 | N | Bayer CropScience |
| KCP 5.2/1 | Guillet, M., Venet, C. & Simonin, B., | 1995 | Isoxaflutole and Metabolites: Analytical Method for the Determination of Residues in Soil. Generated by: Rhône-Poulenc Secteur Agro, Lyon, France Submitted by: Rhône-Poulenc Agro, France Report/file N°: AR 106-94 (E), Docmap n° 437490, Date of report: February 2, 1995. | N | Bayer CropScience |
| KCP 5.2/2 | Schuster, L.L., | 1995 | Method Confirmation for RPA 201772 and its Metabolites RPA 202248, RPA 203328 and RPA 205834 in Soil. Generated by: ABC Laboratories, Pan-Ag Division, Madera, California, USA Submitted by: Rhône-Poulenc Agro, France Report/file N°: EC-95-305 Date of report: November 7, 1995 | N | Bayer CropScience |
| KCP 5.2/3 | Plaisance, R.S., 1995 | 1995 | Validation of Method of Analysis for Isoxaflutole and its Metabolites in/on Agricultural Soil. Generated by: Rhône-Poulenc Ag, Co., USA. Submitted by: Rhône-Poulenc Agro, France. Report/file N°: EC-95-299 Date of report: November 6, 1995 | N | Bayer CropScience |
| KCP 5.2/4 | Corgier, M. and Turier, G., | 1995 | Analytical Method for the Determination of Isoxaflutole in Air. Generated by: Rhône-Poulenc Agro, France. Submitted by: Rhône-Poulenc Agro, France. Report/file N°: 94-115 Docmap 438832 Date of report: June 15, 1995 | N | Bayer CropScience |
| KCP 5.2/5 | Guillet, M. , Diot, R. & Le Gren, I., | 1995b | Isoxaflutole and/or Metabolites: Analytical Method for the Determination of Residues in Animal Products Generated by: Rhône-Poulenc Agro, France Submitted by: Rhône-Poulenc Agro, France Report/file N°: AR 109-95 (E) Date of report: April 12, 1995 | N | Bayer CropScience |
| KCP 5.2/6 | Guillet, M. , & Le Gren, I., | 1995d | Isoxaflutole and/or Metabolites: Analytical Method for the Determination of Residues in Animal Products Complementary Report Generated by: Rhône-Poulenc Agro, France Submitted by: Rhône-Poulenc Agro, France Report/file N°: 9515587 Date of report: May 23, 1995 | N | Bayer CropScience |
| KCP 5.2/7 | Guillet, M. , Diot, R. & Le Gren, I., | 1995b | Isoxaflutole and/or Metabolites: Analytical Method for the Determination of Residues in Animal Products Complementary Report n°:2 Generated by: Rhône-Poulenc Agro, France Submitted by: Rhône-Poulenc Agro, France Report/file N°: 9516279 Date of report: September 20, 1995 | N | Bayer CropScience |
| KCP 5.2/8 | Hunt, T.W. and Lopes, A., | 1995 | Isoxaflutole - Validation of Method of Analysis for Isoxaflutole and its Metabolites in Animal Tissues. Generated by: Rhône-Poulenc Ag Co., USA Submitted by: Rhône-Poulenc Agro, France Report/file N°: EC-95-313 Date of report: December 28, 1995 | N | Bayer CropScience |
| KCP 5.2/9 | Shaffer, S. | 1995 | Independent Laboratory Validation of the Rhône-Poulenc Methods entitled, "Method of Analysis for the determination of Isoxaflutole and its Metabolites (RPA 203328, RPA 202248 and RPA 205834 in Milk" and "Method of Determination for Isoxaflutole and its Metabolite (RPA 202248) in/on Bovine and Poultry Tissues" Generated by: Horizon Laboratories Inc. Columbia, Missouri, USA Submitted by: Rhône-Poulenc Agro, France Report/file N°: EC-95-233 Date of report: December 22, 1995 | N | Bayer CropScience |

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

List of data relied on not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |

- Detailed evaluation of submitted analytical methods

  - Analytical methods for isoxaflutole

    - Methods used for the generation of pre-authorization data (KCP 5.1)

No new or additional studies have been submitted

    - Methods for post-authorization control and monitoring purposes (KCP 5.2)

      - Description of analytical methods for the determination of residues in plant matrices (KCP 5.2)

No new or additional studies have been submitted

      - Description of analytical methods for the determination of residues in animal matrices (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Soil (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Water (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Air (KCP 5.2)

No new or additional studies have been submitted

      - Description of Methods for the Analysis of Body Fluids and Tissues (KCP 5.2)

No new or additional studies have been submitted

      - A.2.A.9	Other Studies/ Information

No new or additional studies have been submitted