REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: AF-777 Product name(s): SmartFreshTM ProTabs Chemical active substance: 1-Methylcyclopropene (1-MCP), 20 g/kg (2.0 %)

Interzonal Interzonal Rapporteur Member State: The Netherlands (NL)

CORE ASSESSMENT (re-authorisation according to Article 43)

Applicant: AgroFresh Holding France S.A.S. Submission date: 28.10.2019 MS Finalisation date: March 2023

Table of Contents

1	Section 1: Identity of the plant protection product	5

1.1	Applicant (KCP 1.1)	5

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	5

1.2.1	Producer(s) of the preparation	5

1.2.2	Producer(s) of the active substance(s)	5

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	5

1.2.3.1	1-Methylcyclopropene	5

1.2.3.2	Active substance 2	6

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	6

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	6

1.4.1	Composition of the plant protection product (KCP 1.4.1)	6

1.4.2	Information on the active substance(s) (KCP 1.4.2)	7

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	7

1.5	Type and code of the plant protection product (KCP 1.5)	7

1.6	Function (KCP 1.6)	7

2	Section 2: Physical, chemical and technical properties of the plant protection product	8

3	Section 3 is presented as a separate document	39

4	Section 4: Further information on the plant protection product	39

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	39

Appendix 1	Lists of data considered in support of the evaluation	41

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	53

A 2.1	1-Methylcyclopropene	53

A 2.2	Active substance 2	53

Introduction

This document supports the application for the renewal of the authorization (according to Article 43 of Regulation (EC) No 1107/2009) and summarises the information related to the identity, the physical and chemical properties, the data on application, further information and the classification for the formulation AF-777, Tablets formulation (TB) containing 2.0% of 1-methylcyclopropene (1-MCP), already registered in different EU countries under the name of SmartFreshTM ProTabs. AF-777 is to be used indoors, as a post-harvest plant growth regulator in airtight areas. The intended uses for EU range of crops are apples, pears, plums, tomatoes, kiwifruits, persimmons, peaches, nectarines, apricots, pomegranates and melons.

AF-777 is the product code which is used in all sections of the dossier. The registered product name remains SmartFreshTM ProTabs.

1-MCP was included into Annex I of Directive 91/414 (Regulation 2006/19/EC) and its approval is renewed in the AIR procedure (Reg. (EU) 2019/1085).

Article 43 submission

The dRR follows as much as possible the current product authorisation. However, the original RR has been revised at some points as a result of the following new parameters:

-	New specification as a result of the renewal process (purity and impurities of a.s.)

-	New studies

-	A new packaging size

No amendment of the original authorised GAP is foreseen for the application of re-authorisation for the product AF-777 in accordance with Article 43 of Regulation (EC) 1107/2009.

Where appropriate this document refers to the conclusions of the EU review of 1-MCP. This will be where:

- the active substance data is relied upon in the risk assessment of the formulation

- the EU review concluded that additional data/information should be considered at national re-registration.

The SANCO report for 1-Methylcylcopropene (SANCO/2005/1094 - rev. 6), the EFSA scientific reports (EFSA 2005; 30, 1-46 and EFSA 2018;16(7):5308) and the renewal report (SANTE/11631/2018 Rev 5 - 05.05.2019) are considered to provide the relevant review information or a reference to where such information can be found. The relevant EU agreed endpoints to be used in this evaluation are included in the respective Annex points of this section.

The following table provides the EU endpoints to be used in the evaluation of this section.

Agreed EU End-points (Renewal report SANTE/11631/2018 Rev 5)

| End-Point | Content |
| --- | --- |
| Minimum purity of 1-MCP | 980 g/kg |
| Identity of relevant impurities (of toxicological, environmental and/or other significance) in 1-MCP as manufactured | 1-CMP: ≤ 0.2 g/kg 3-CMP: ≤ 0.2 g/kg |

1-CMP = 1-Chloro-2-methylpropene; 3-CMP = 3-Chloro-2-methylpropene

The Annex I Implementing regulation for 1-MCP (Reg. (EU) No 2019/1085) provides specific provisions under Part B which need to be considered by the applicant in the preparation of their submission and by the MS prior to granting an authorisation.

For the implementation of the uniform principles of Annex VI, the conclusions of the renewal report (SANTE/11631/2018 Rev 5) on 1-Methylcylcopropene, and in particular Appendices I and II thereof, as finalised in the Standing Committee on the Food Chain and Animal Health on 21 May 2019 shall be taken into account. In this overall assessment:

On the basis of the proposed and supported uses, no particular issues have been identified as requiring short term attention from the Member States.

Information on the detailed composition of AF-777 can be found in the confidential dossier of this submission (Registration Report - Part C).

Note: this Part B document only reviews data (Annex II/CA or Annex III/CP) and additional information that has not previously been considered within the EU review process, as part of the renewal decision.   New annex II/CA data must only be included if they are considered essential for the evaluation and in this case a full study summary must be provided.

This product was not the representative formulation in the Annex I renewal, but, this product has recently been evaluated in several European Member States according to Uniform Principles (see table 1.0-1).

Table 1.1-1:	List of the countries, where SmartFreshTM ProTabs is currently registered

| Product code | Product name(s) | MS | Authorization No. | Date of initial registration | Date of the last re-registration |
| --- | --- | --- | --- | --- | --- |
| AF-777 | SmartFreshTM ProTabs | Austria | 3651 | 10.12.2015 | --- |
| AF-777 | SmartFreshTM ProTabs | Belgium | 10414P/B | 09.03.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Bulgaria | 01621-ПРЗ-1/13.09.2018 г. | 13.09.2018 | --- |
| AF-777 | SmartFreshTM ProTabs | Croatia | UP/I-320-20/15-01/344 | 12.04.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Czech Republic | 5399-0 | 02.06.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | France | 2150057 | 29.06.2015 | --- |
| AF-777 | SmartFreshTM ProTabs | Germany | 008368-00 | 30.03.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Greece | 8253/31-5-2016 | 31.05.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Hungary | 04.2/1181-3/2017 | 30.06.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Ireland | 05779 | 05.02.2018 | --- |
| AF-777 | SmartFreshTM ProTabs | Italy | 16244 | 09.07.2015 | --- |
| AF-777 | SmartFreshTM ProTabs | Luxembourg | L02191-181 | 08.03.2018 | --- |
| AF-777 | SmartFreshTM ProTabs | Netherlands | 14648N | 31.10.2014 | --- |
| AF-777 | SmartFreshTM ProTabs | Poland | R-27/2016 | 04.11.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Portugal | 00962 | 27.06.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Romania | 380PC | 29.11.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Slovakia | 16-14-1840 | 20.10.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | Slovenia | U34330-102/2016/3 | 26.10.2017 | --- |
| AF-777 | SmartFreshTM ProTabs | Spain | ES-00179 | 30.03.2016 | --- |
| AF-777 | SmartFreshTM ProTabs | UK | 16546 | 08.07.2014 | --- |

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name:	AgroFresh Holding France S.A.S.

Address:	23-25, avenue Mac-Mahon

75017 Paris

France

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### 1-Methylcyclopropene

CONFIDENTIAL information - data provided separately (Part C).

| 1-MCP | min. 980 g/kg |
| --- | --- |

| 1-Chloro-2-methylpropene (1-CMP) | Max. 0.2 g/kg in the technical material |
| --- | --- |
| 3-Chloro-2-methylpropene (3-CMP) | Max. 0.2 g/kg in the technical material |

#### Active substance 2

There is no second active substance in the plant protection product.

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | SmartFreshTM ProTabs |
| --- | --- |
| Company code number: | AF-777 |

## Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

Table 1.4-2:	1-MCP and variant(s) of 1-MCP

| Active substance / variant | Declared content of the pure active substance / variant (g/L or g/kg) | FAO Limits (min – max) | Technical content* (g/L or g/kg) | Technical content (%w/w) |
| --- | --- | --- | --- | --- |
| 1-Methylcycloprope-ne (in an encapsulated α-cyclodextrin complex) | 20 g/kg | ± 25% of the declared content | 980 g/kg | > 98% technical |

* 	Based on the minimum purity of the active substance declared for registration in the active substance dossiers

|  | Nominal content | FAO tolerance |
| --- | --- | --- |
| Pure active ingredient 1-methylcyclopropene | 20 g/kg (2%w/w) | 15 – 25 g/kg |

With a minimum purity of 980 g/kg

Table 1.4-3:	Safener and synergists

Confidential information, please refer to Part C.

Table 1.4-4:	Relevant impurities

| Relevant impurity | Maximum content (g/kg)* |
| --- | --- |
| 1-chloro-2-methylpropene (1-CMP) | ≤ 0.2 g/kg |
| 3-chloro-2-methylpropene (3-CMP) | ≤ 0.2 g/kg |

*Maximum content in the TC.

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4-5:	Information on active substance

| Type | Name/Code Number |  |
| --- | --- | --- |
| ISO common name | 1-MCP | Variant: not allocated |
| CAS No. | 3100-04-7 |  |
| EC No. | - |  |
| CIPAC No. | 767 |  |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information - data provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: tablet product | Code: TB |
| --- | --- |

## Function (KCP 1.6)

The formulation AF-777 is a indoors post-harvest plant growth regulator.

# Section 2: Physical, chemical and technical properties of the plant protection product

SmartFreshTM ProTabs (AF-777) was not the representative formulation for the renewal of approval of the active substance 1-MCP. However, the product has already been authorized in various EU countries. Most of the studies presented to support the physical, chemical and technical properties of the produce have therefore been evaluated by the zRMS UK for the previous authorization (UK, 2014). The following data gaps were identified in the RR for the original product authorization:

- Shelf life study at ambient temperature in the commercial packaging material.

- A test to determine the percentage release of 1-MCP from SmartFreshTM ProTabs

- The determination of the content of the relevant impurities before storage

New data have been submitted within the framework of this application to address the above mentioned data gaps. For completeness, the physical, chemical and technical properties of the formulation is discussed below based on the results from all available studies.

The product comprises three separate components: SmartFreshTM ProTabs (consisting yellow or pink tablets), SmartFresh™ ProPack (activator solution) and SmartFresh™ ProActivators (activator tablets ). The active ingredient, 1-MCP, is released when both tablets (SmartFreshTM ProTabs and Smart-Fresh™ ProActivators) are added to SmartFresh™ ProPack. The relevant physical, chemical and technical properties of each component were assessed in the studies and will be summarized below.

SmartFreshTM ProTabs:

SmartFreshTM ProTabs contain 1-MCP at nominal a concentration of 2.0%. The tablets as manufactured contain two colures: SmartFreshTM ProTab pink tablets for smaller volumes (50 - 250 m3) and SmartFreshTM ProTab yellow tablets for larger volumes (250 – 3 000 m3).

SmartFreshTM ProTabs pink and yellow tablets:

The SmartFreshTM ProTab pink tables are pink in colour and weigh 0.84 grams while the SmartFreshTM ProTab yellow tablets are yellow in colour and weigh 4.2 grams. The formulation has a sweet odour. It is not explosive or oxidising and is not considered to be highly flammable. The auto-flammability was determined to be 221oC. In aqueous solution (1%) it has a pH value of 8.38 at 22°C. For SmartFresh ProTabs yellow tablets, the average pH was 8.18 before storage and 8.23 after storage for 14 days at 54°C. For SmartFresh ProTabs pink tablets, the average pH was 8.15 before storage and 8.19 after storage for 14 days at 54°C.

The pink tablets have a pour density of 0.8805 g/mL and tap density of 0.8925 g/mL. The yellow tablets have a pour density of 0.7924 g/mL and tap density of 0.8050 g/mL.

The storages stability test at accelerated temperature showed that the SmartFreshTM ProTabs are heat insensitive when stored in plastic foil pouch for 2 weeks at 54oC. Furthermore, the shelf life study showed that the SmartFreshTM ProTabs  are stable for two years when stored at ambient temperature in commercial packaging (plastic foil pouch). No changes were observed in the content of the active substance and the weight and appearance of the packaging material. After storage for two years, the release rate of the tablets was above 90%. The content of the relevant impurities 1-chloro-2-methyl propene and 3-chloro-2-methyl propene has been determined before storage. The content of the impurities was below the limit of detection (LOD  < 0.015%). SmartFreshTM ProTabs are considered to be stable for two years at ambient temperature when stored in plastic foil pouch.

The spontaneity of dispersion of the pink tables was 26 minutes at the lowest (1 tablet) and 32 minutes at the highest application rate (10 tablets). While the spontaneity of dispersion of the yellow tables was 56 minutes at the lowest application rate (1 tablet) and 59 minutes at the highest application rate (20 tablets). The attrition resistance of the tables was above 99%. The tablet integrity has been demonstrated to be maintained.

SmartFresh ProActivator tablets:

The appearance of the SmartFresh ProActivator tablets are that of an off-white tablet with no odour. The formulation is not explosive or oxidising and is not considered to be highly flammable. The auto-flammability was determined to be >400oC. In aqueous (1%) solution it has a pH value of 8.26 at 25°C. The tablets have a pour density of 1.15 g/mL g/mL and tap density of 1.16 g/mL.

The integrity of the  SmartFresh™ ProActivators pouch (PET/PE/Foil/PE) was demonstrated after storage for two weeks at 54 oC. The integrity of the pouches for two years at ambient temperature can be granted by extrapolation form the data of the accelerated stability study.

The spontaneity of dispersion of the tables was 2 hours at the lowest (8 tablet) and 4 hours at the highest application rate (40 tablets). The attrition resistance of the tables was above 100%. Tablets were in good condition with no broken tablets found.

SmartFresh ProPack – Activator Solution:

The appearance of the SmartFresh ProPack – Activator Solution is that of a clear liquid with no odour. The formulation is not explosive or oxidising. The flash point of the formulation was not observed up to the boiling point at 105 oC. The formulation is not self-igniting as the auto-flammability temperature was above 400oC. In aqueous solution (1%) it has a pH value of 2.81 with an acidity of 7.31% w/w (calculated as H2SO4). Based on the pH of the formulation and the fact that H290 classified component exists in the composition, a study was requested to confirm if corrosiveness to metals can be avoided. According to the applicant, no study for corrosiveness to metals is required on the basis of the composition of the activator solution. Instead a position paper has been presented demonstrating that the H290 classified ingredient (sodium hydroxide) is only corrosive under strongly basic conditions. However, as a result of the presence of an excessive acidic component in the solution, a neutralization reaction occurs whereby the corrosive property of the solution can be excluded. The position paper is accepted. Classification as may be corrosive to metals (H290) can be avoided on the basis of the composition of the activator solution.

The surface tension tested at 0.1 dilution was 43.9  mN/m at 20 oC. The product is surface active. The kinematic viscosity at 40 oC was 0.95 mm2/s. Classification as aspiration hazard is not triggered since no H304 classified components present in the composition. It has a density of 1.0378 g/ml.

The activator solution was found to be stable following 7 days storage at 0oC. No change was shown in the commercial packaging after storage of the SmartFresh ProPack Solution for 2 weeks at 54°C.

Overall conclusion regarding the product’s storage stability

Based on the available data, zRMS assigns a provisional 2-year shelf-life in the proposed commercial packaging. The current data package lacks the required data on post long-term storage pH for the ProPacks

and post long-term data on packaging integrity of the ProActivators and ProPacks. Given the low immediacy of these shortcomings, zRMS deems that the mentioned data may be submitted post-authorization to finalize the storage stability assessment.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

No Classification proposed for physical chemical part.

Notifier Proposals for Risk and Safety Phrases (KCP 12)

No Risk and Safety Phrases proposed for physical chemical part.

Compliance with FAO specifications:

No FAO specification available for products containing 1-MCP.

Formulation used for tests

The product used in the tests has the same composition as the one cited in Part C.

Table 2-6:	Physical, chemical and technical properties of the plant protection product

| Test or study & Annex point | Method used / deviations | Test material purity and specification | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | EPA/OPPTS 830.6302, 830.6303, 830.6304 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. RMJ8020C a.s. content: 2.0% w/w 1-MCP | AF-777 1-MCP Formulation pink: Pink tablets (weight: 0.84 g). Sweet odour. | Y | Kühne, M. (2011h) Report No. AF-11-021 | Acceptable |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | AF-777 1-MCP Formulation yellow: Yellow tablets (weight: 4.2 g). Sweet odour. | Y | Kühne, M. (2011l) Report No. AF-11-028 | Acceptable |
|  |  | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 composition: 10% citric acid 2.64% Arquad C33W 0.40% sodium hydroxide 0.30% AntiFoam | AF-777 Activator Solution is a clear colourless liquid without any odour. | Y | Kühne, M. (2011t) Report No. AF-11-047 | Acceptable |
|  |  | SmartFresh™ ProActivators Batch-No.711V3PVP | ProActivators Tablets are cream/off-white opaque solid tablet. Odourless. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 | Acceptable |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | AF-777 Spent Activator Solution is a turbid yellowish liquid containing some white flocks. Weak and sweet odour. | Y | Kühne, M. (2011a) Report No. AF-11-011 | Acceptable |
| Explosive properties (KCP 2.2.1) | EPA/OPPTS 830.6316, UN Recommendations of the Transport of Dangerous Goods, Orange Book, 4th ed. United Nations 2003 | SmartFreshTM ProTabs Batch-No. RMJ8019A a.s. content: 1.90% w/w 1-MCP | Expert statement: Not to be classified as explosive material. None of the ingredients of AF-777 is labelled “explosive” according to EC Directives. Thus, none of the components of AF-777 (system consisting of AF-777 Yellow or Pink Tablets, SmartFresh™ ProActivators and, SmartFresh™ ProPack is explosive. | - | Verona, D. (2011g) Report No. AF-11-068 | Acceptable The formulation is considered to be not explosive in the sense of CLP Reg. (EC) 1272/2008, based on a theoretical assessment of the structure of the active substance and all components in the composition. |
| Oxidizing properties (KCP 2.2.2) | EPA/OPPTS 830.6314 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | AF-777 1-MCP Formulation yellow was found to be chemically compatible with the following reagents: water, oxidizing agent (aqueous sodium chlorate solution) and reducing agent (zinc dust). The test item was incompatible with fire extinguishing agent (mono-ammonium phosphate). Thus, the test substance neither has oxidising nor reducing properties but shows chemical incompatibility to fire extinguishing agent (mono-ammonium phosphate) in the sense of EPA method OPPTS 830.6314. | Y | Kühne, M. (2011m) Report No. AF-11-029 | Acceptable The formulation is considered to be not oxidising in the sense of CLP Reg. (EC) 1272/2008, based on a theoretical assessment of the structure of the active substance and all components in the composition. |
|  | EPA/OPPTS 830.6314 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | AF-777 Activator Solution was found to be chemically compatible with the following reagents: water, oxidizing agent (aqueous sodium chlorate solution), reducing agent (zinc dust) and fire extinguisher chemical (mono-ammonium phosphate). Thus, the test substance neither has oxidising nor reducing properties nor shows chemical incompatibility to any of the tested reagents in the sense of EPA method OPPTS 830.6314. | Y | Kühne, M. (2011u) Report No. AF-11-048 |  |
|  | EPA/OPPTS 830.6314 | SmartFresh™ ProActivators Batch-No.711V3PVP | Oxidizing properties have been predicted negative based on the chemical structures of the test item. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
|  | EPA/OPPTS 830.6316, UN Recomendations of the Transport of Dangerous Goods, Orange Book, 4th ed. United Nations 2003 | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | AF-777 Spent Activator Solution was found to be chemically compatible with the following reagents: water, oxidizing agent (aqueous sodium chlorate solution), reducing agent (zinc dust) and fire extinguisher chemical (mono-ammonium phosphate). Thus, the test substance neither has oxidising nor reducing properties nor shows chemical incompatibility to any of the tested reagents in the sense of EPA method OPPTS 830.6314. | Y | Kühne, M. (2011b) Report No. AF-11-012 |  |
| Flash point (KCP 2.3.1) | 92/69/EEC A.9 440/2008/EC A.9 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | AF-777 Activator Solution was heated from 25°C up to a maximum temperature of 105°C. The test item started boiling at 101°C. No flash point was observed in the temperature range between 25°C and 105°C. | Y | Kühne, M. (2011v) Report No. AF-11-049 | Acceptable Appropriate validated method EEC A.9 was used. No flash point was observed. The test substance was observed to boil at approximately 101 oC. |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | AF-777 Spent Activator Solution was heated from 25°C up to a maximum temperature of 105°C. The test item started boiling at 102.5°C. No flash point was observed in the temperature range between 25°C and 105°C. | Y | Kühne, M. (2011c) Report No. AF-11-013 | Acceptable Appropriate validated method EEC A.9 was used. No flash point was observed. The test substance was observed to boil at approximately 105 oC. |
| Flammability (KCP 2.3.2) | 92/69/EEC A.10 440/2008/EC A.10 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | AF-777 1-MCP Formulation yellow could not be ignited with a flame within 2 minutes. Therefore, the test item is not highly flammable according to the specified guidelines. As the composition is identical, only the dye is different, this is also valid for AF-777 (pink tablet). | Y | Kühne, M. (2011n) Report No. AF-11-030 | Acceptable Appropriate validated method EEC A.10 was used. The formulation is not highly flammable in the sense of CLP Reg. (EC) 1272/2008. |
|  |  | SmartFresh™ ProActivators Batch-No.711V3PVP | The test item is not highly flammable according to the specified guidelines. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
| Self-heating (KCP 2.3.3) | 92/69/EEC A.16 440/2008/EC A.16 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | AF-777 1-MCP Formulation yellow shows a self-ignition temperature at about 221°C. Therefore, the test item is to be considered as auto-flammable. As the composition is identical, only the dye is different, this is also valid for SmartFreshTM ProTabs (pink tablet). | Y | Kühne, M. (2011o) Report No. AF-11-031 | Acceptable All the components of the formulation are considered not self-heating in the sense of CLP Reg. (EC) 1272/2008 based on a test with EEC A. 16. |
|  | 92/69/EEC A.16 440/2008/EC A.16 | SmartFresh™ ProActivators Batch-No.711V3PVP | No relative self-ignition has been determined below 400°C. Therefore, the test item is not to be considered as auto-flammable. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
|  | 440/2008/EC A.15 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | AF-777 Activator Solution has been determined not to have an auto-ignition temperature below 400°C. Therefore, the test item is not to be considered as auto-flammable. | Y | Atwal, S.S. et al. (2011b) Report No. AF-11-046 |  |
|  | 440/2008/EC A.15 | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | AF-777 Spent Activator Solution has been determined not to have an auto-ignition temperature below 400°C. Therefore, the test item is not to be considered as auto-flammable. | Y | Atwal, S.S. et al. (2011a) Report No. AF-11-045 |  |
| Acidity or alkalinity and pH (KCP 2.4.1) | CIPAC MT 191 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | Not required as pH is between 4 and 10. Please refer to MIIIA1 2.4.2 |  |  | Acceptable |
|  |  | SmartFresh™ ProActivators Batch-No.711V3PVP | Not required as pH is between 4 and 10. Please refer to MIIIA1 2.4.2 | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
|  |  | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | The acidity of AF-777 Activator Solution was determined to be 7.31% w/w (calculated as H2SO4 according to CIPAC method MT 191) | Y | Kühne, M. (2011w) Report No. AF-11-050 |  |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | Not required as pH is between 4 and 10. Please refer to MIIIA1 2.4.2 |  |  |  |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | pH = 8.38 at 22°C. | Y | Kühne, M. (2011p) Report No. AF-11-032 | Acceptable |
|  |  | SmartFresh™ ProActivators Batch-No.711V3PVP | pH = 8.26 at 25°C. |  | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
|  |  | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | pH = 2.81 at 20°C. Based on the detailed composition of the solution SmartFresh ProPack, it is shown that no study is necessary, as the H290 classified ingredient (sodium hydroxide) is only corrosive under strongly basic conditions which are not given in the acidic mixture. Additionally, there is a neutralization reaction with the pre-sent excess of citric acid, the products of which are not classified. | Y | Kühne, M. (2011w) Report No. AF-11-050 | Acceptable Based on the pH of the formulation and the fact that H290 classified component exists in the composition triggers a study on corrosiveness to metals. However, in addition to the H290 classified component (sodium hydroxide , basic), the solution contains also an excessive amount of acid component. Consequently, a neutralization reaction occurs whereby the corrosiveness property of the solution can be excluded. |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | pH = 5.50 at 23°C. | Y | Kühne, M. (2011d) Report No. AF-11-014 | Acceptable |
| Viscosity (Kinematic) (KCP 2.5.1) | OECD Guideline No. 114 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | AF-777 Activator Solution has a kinematic viscosity of 1.45 mm2/s at 20°C and 0.95 mm2/s at 40°C. | Y | Kühne, M. (2011x) Report No. AF-11-051 | Acceptable The kinematic viscosity is 0.95 mm2/s at 40°C which is below the threshold of 20.5 mm2/s for classification as an aspiration hazard. However, no classification as aspiration hazard is required for the product since no H304 classified components present in the composition. |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | Not applicable for AF-777 Spent Activator Solution, as measurements with a capillary viscometer failed due to clogging by solid content of the test item. | Y | Kühne, M. (2011e) Report No. AF-11-015 | Acceptable |
| Viscosity (Dynamic) (KCP 2.5.1) | OECD Guideline No. 114 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | AF-777 Activator Solution has a dynamic viscosity of 1.50 mPa s at 20°C. | Y | Kühne, M. (2011x) Report No. AF-11-051 | Acceptable According to the data requirement, the dynamic viscosity should also be determined at different shear rates and 40°C. However, considering no H304 classified components present in the composition no risk for classification is expected for the product. Therefore, no study was requested for the intended product authorization. |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | AF-777 Spent Activator Solution is a Newtonian fluid, the viscosity values did not vary with the shear rate. The test item has a dynamic viscosity of 4.22 mPa s at 20°C and 2.65 mPa s at 40°C. | Y | Kühne, M. (2011e) Report No. AF-11-015 | Acceptable The kinematic viscosity is 2.65 mm2/s at 40°C which is below the threshold of 20.5 mm2/s for classification as an aspiration hazard. However, no classification as aspiration hazard is required for the product since no H304 classified components present in the composition. |
| Surface tension (KCP 2.5.2) | 92/69/EEC A.5 OECD Guideline No. 115 440/2008/EC A.5 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | The surface tension of AF-777 Activator Solution in water (at a concentration of 1 g/L) was determined to be 43.9 mN/m at 20.2°C. | Y | Kühne, M. (2011y) Report No. AF-11-052 | Acceptable The formulation is surface active. |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | The surface tension of AF-777 Spent Activator Solution in water (at a concentration of 1 g/L) was determined to be 37.7 mN/m at 20.3°C. | Y | Kühne, M. (2011f) Report No. AF-11-016 |  |
| Relative density (KCP 2.6.1) | 92/69/EEC A.3 OECD Guideline No. 109 440/2008/EC A.3 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | The relative density of AF-777 Activator Solution was determined to be D420 = 1.0378. | Y | Kühne, M. (2011z) Report No. AF-11-053 | Acceptable |
|  |  | Spent ProPack Solution (AF-777 Spent Activator Solution) Batch-No. FLW82:1D | The relative density of AF-777 Spent Activator Solution was determined to be D420 = 1.0985. | Y | Kühne, M. (2011g) Report No. AF-11-017 |  |
| Bulk or tap density (MIIIA1 2.6.2) | CIPAC MT 186 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. RMJ8020C a.s. content: 2.0% w/w 1-MCP | Pour density: 0.8805 g/mL Tap density: 0.8925 g/mL | Y | Kühne, M. (2011i) Report No. AF-11-022 | Acceptable |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | Pour density: 0.7924 g/mL Tap density: 0.8050 g/mL | Y | Kühne, M. (2011q) Report No. AF-11-033 |  |
|  | CIPAC MT 186 | SmartFresh™ ProActivators Batch-No.711V3PVP | Pour density: 1.15 g/mL Tap density: 1.16 g/mL | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | EPA/OPPTS 830.6313, 830.6320 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. RMJ8020C a.s. content: 2.0% w/w 1-MCP | Tablets were packed in plastic lined foil pouches. Pouch content: 1 AF-777 pink tablets and 8 activator tablets. Pouch weight: Before storage: 25.86 g After storage: 25.77 g Appearance, packaging integrity and corrosion: Before and after storage: No change in appearance was observed after storage. There was no observable deterioration in the surfaces of the pouches, and the integrity of the pouch seals was maintained. a.s. content: Before storage: 2.18% w/w 1-MCP After storage: 1.99% w/w 1-MCP 1-MCP change: -8.7% 1-MCP release characteristics: 12% decrease in 1-MCP release after storage at 54°C for 14 days. Release profiles were shown to be comparable before and after storage. | Y | Verona, D. (2011e) Report No. AF-11-102 Verona, D. (2011f) Report No. AF-11-134 | Acceptable The content of the active substance decreased with >5% after storage for two weeks at 54 oC. According to the FAO/WHO Manual if the active substance concentration decreases by more than 5% in comparison with the initially measure value, then information about possible degradation products should be provided. No degradation products are expected from the intended product. The content of the impurities was determined before storage (KCP 2.11). Both of the impurities were determined below the limit of detection of 0.015%. As the impurities are process impurities, their formation on storage is not expected. The integrity of the SmartFreshTM ProTabs pouch (pink and yellow), SmartFresh™ ProActiva-tors pouch and Smart-Fresh™ ProPack stand up pouch was demonstrated after storage for two weeks at 54 oC. The formulation SmartFreshTM ProTabs (pink and yellow) is considered stable for two weeks at 54 oC in pouches. The GC/FID method validated in study AF-11-067 was used for the determination of the content of 1-MCP in the formula-tion. The method is validated in accord-ance with SAN-CO/3030/99 rev.4 in B5, KCP 5.1.1/01. Upon request, the applicant has amended the dossier with data on disintegration and attrition. Disintegration time was shown to remain within practical limits (i.e., the tablets are fully disintegrated well within the treatment time). The information provided as placeholder for the post-storage attrition endpoint cannot be considered useful in any way, as it does not tell anything about the behavior of the post-storage tablets when they are actually subjected to friction force. As (i) the tablets are packed separately and the operators are not likely exposed to any dust produced by the tablets during use, and (ii) the tablets are packed in a moisture-proof packaging and have an excellent innate fraction resistance, this is nevertheless accepted. |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | Tablets were packed in plastic lined foil pouches. Pouch content: 10 AF-777 yellow tablets and 40 activator tablets. Pouch weight: Before storage: 109.50 g After storage: 110.04 g Appearance, packaging integrity and corrosion: Before and after storage: No change in appearance was observed after storage. There was no observable deterioration in the surfaces of the pouches, and the integrity of the pouch seals was maintained. No change in the commercial packaging was seen after storage for both components. a.s. content: Before storage: 2.06% w/w 1-MCP After storage: 1.87% w/w 1-MCP 1-MCP change: -9.3% 1-MCP release characteristics: 8% decrease in 1-MCP release after storage at 54°C for 14 days. Release profiles were shown to be comparable before and after storage. pH Yellow Tablets: Before storage: 8.18 After 14 days at 54°C: 8.23 pH Pink Tablets: Before storage: 8.15 After 14d at 54°C: 8.19 Disintegration time Pink Tablets T0; After 14d54°C. 1 Tablet; 45 min; 3 hours. 25 Tabs.; 40 min; 2 hours 45 min. Disintegration time Yellow Tablets T0; After 14d54°C. 1 Tablet; 60 min; 60 min. 20 Tabs.; 3 hrs 45 min.; 3 hrs 45 min.. There is no significant changes in disintegration time for both pink and yellow SmartFresh ProTabs tablets. Attrition resistance: Coating tablets are not susceptible to attrition as the coating is designed to prevent it. Less than 0.02% was lost after storage for both pink and yellow SmartFresh ProTabs tablets. SMARTFRESH PROPACK: Packaging integrity: A small average weight loss of 0.73% was noted before and after storage.; No change in the commercial packaging was seen after storage. pH before storage = 2.39 After 14 days at 54°C: 2.38 Acidity before storage = 6.25 Acidity after storage = 6.15 SMARTFRESH PROACTIVATORS: Packaging integrity: No change in appearance was observed after storage with < 0.05% weight loss after storage, a slight change was noted for the color from light cream to light yellow. No change in the commercial packaging was seen after storage. Attrition resistance: Coating tablets are not susceptible to attrition as the coating is designed to prevent it, No difference in weight between the aged and unaged ProActivators tablets was noted. pH SmartFresh ProActivators: Before storage: 8.30 After 14 days at 54°C: 8.41 Desintegration time for aged SmartFresh ProActivators and SmartFresh Propack: Disintegration time before storage = 237 min. Disintegration time after 14 days at 54°C= 283 min. It’s a non significant increase knowing that a duration of 24 hours is required for the application. | Y | Verona, D. (2011e) Report No. AF-11-102 Verona, D. (2011f) Report No. AF-11-134 F. Wehmeyer (2016) Report No. AF-15-109 F. Wehmeyer (2017) Report AF-17-057 F. Wehmeyer (2017) Report AF-17-056 F. Wehmeyer (2016) Report No. AF-15-109 F.Wehnmeyer (2022) Report No. AF-22-007 |  |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | OPPTS Test Guidelines,.series 830 830.6317 Storage Stability 830.6320 Corrosion Characteristics (after 12 months at 25°C) | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. VEC298-2011 Lot 1 a.s. content: 2.0% w/w 1-MCP | Pouch weight: Before storage: 110.37 g After storage: 110.39 g Appearance, packaging integrity and corrosion: Before and after storage: Visual inspection of the pouches showed they were not compromised and both the inner and outer surfaces of the foil pouches were unremarkable. a.s. content: Before storage: 2.1% w/w 1-MCP After storage: 2.0% w/w 1-MCP | Y | Kurilla, K. (2013) Report No. AF-13-005 | Acceptable See above (KCP 2.7.1) for the additional discussion. |
|  | OPPTS Test Guidelines,.series 830 830.6317 Storage Stability 830.6320 Corrosion Characteristics (after 12 months at 25°C) | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. VEC294-2011 Lot 2 a.s. content: 2.0% w/w 1-MCP | Pouch weight: Before storage: 26.44 g After storage: 26.47 g Appearance, packaging integrity and corrosion: Before and after storage: Visual inspection of the pouches showed they were not compromised and both the inner and outer surfaces of the foil pouches were unremarkable. a.s. content: Before storage: 2.2% w/w 1-MCP After storage: 2.1% w/w 1-MCP | Y | Kurilla, K. (2013) Report No. AF-13-007 |  |
| Minimum content after heat stability testing (KCP 2.7.3) | nr | nr | Please refer to MIIIA1 2.7.1 | - | - | - |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 | SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:51 | No separated material or in-homogeneity was noted during storage of AF-777 Activator Solution for 7 days at 0°C. Thus, the test item was found to be stable under the test conditions. | Y | Kühne, M. (2011α) Report No. AF-11-054 | Acceptable |
| Ambient temperature shelf life (KCP 2.7.5) | EPA/ OPPTS Test Guidelines, series 830 830.6317 Storage Stability 830.6320 Corrosion Characteristics | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. VEC294-2011 Lot 2 a.s. content: 2.0% w/w 1-MCP | Pouch weight: Before storage: 26.50 g After storage: 26.50 g Appearance, packaging integrity and corrosion: Before and after storage: No change in appearance was observed after 24 months storage at 25°C. There was no observable deterioration in the surfaces of the pouches, and the integrity of the pouch seals was maintained. a.s. content after 24 months storage at 25°C: Before storage: 2.2% w/w 1-MCP After storage: 2.1% w/w 1-MCP 1-MCP change: -4.5% Release rate: Release rate was similar before and after storage 2 years at ambient temperature of the pink tablet and the activation kit. pH: Before storage: 8.15 After 36 months at 25°C: 8.24 Disintegration time: Before storage: 1 tablet: 26 min 38 sec. 10 tablets: 32 min 14 sec. After storage for 36 months at room temperature: 1 tablet: 33 min 38 sec. 10 tablets: 43 min 41 sec. There is no significant differences in disintegration time after 3 years at room temperature. | Y | Kurilla, K. (2014a) Report No. AF-14-008 Wehmeyer F. (2018) Report No. AF-17-043rev1 | Acceptable The content of the active substance decreased with <5% after storage. The content of the two impurities was determined before storage (KCP 2.11). Both were determined below the limit of detection of 0.015%. Since the impurities are process impurities their formation on storage is not expected. The integrity of the SmartFreshTM ProTabs pouch (pink and yellow) was demonstrated after storage for two years at ambient temperature. However, no information is available regarding the package integrity for the SmartFresh™ ProActivators pouch and SmartFresh™ ProPack stand up pouch. For these, reference is made to test conducted in the accelerated storage stability study (KCP 2.7.1). In addition, no data are available on the pH of the SmartFreshTM ProPack. The pH of the SmartFresh ProActivator tablets was shown to remain stable throughout the 36 month test duration. The GC/FID method validated in study AF-11-067 was used for the determination of the content of 1-MCP in the formulation. The method is validated in accordance with SANCO/3030/99 rev.4 in B5, KCP 5.1.1/01. In conclusion of the preceding, a provisional 2-year shelf-life is granted in commercial packaging. -post long-term storage pH for the ProPacks -post long-term data on packaging integrity of the ProActivators and ProPacks. |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. VEC298-2011 Lot 1 a.s. content: 2.0% w/w 1-MCP | Pouch weight: Before storage: 110.04 g After storage: 110.07 g Appearance, packaging integrity and corrosion: Before and after storage: No change in appearance was observed after 24 months storage at 25°C. There was no observable deterioration in the surfaces of the pouches, and the integrity of the pouch seals was maintained. a.i. content: Before storage: 2.1% w/w 1-MCP After storage: 2.0% w/w 1-MCP 1-MCP change: -4.8% Release rate: Release rate was higher than 80% after storage for 2 years at ambient temperature of the yellow tablet and the activation kit. pH Yellow Tablets: Before storage: 8.18 After 36 months at 25°C: 8.27 Disintegration time after storage for 36 months at room temperature: 1 tablet: 33 min 38 sec. | Y | Kurilla, K. (2014b) Report No. AF-14-006 F. Wehmeyer (2017) Report AF-17-057 F. Wehmeyer (2017) Report AF-17-056 Wehmeyer F. (2018) Report No. AF-17-044rev1 |  |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | nr | nr | Please refer to KCP 2.7.5 | - | - | - |
| Wettability (KCP 2.8.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Persistence of foaming (KCP 2.8.2) | nr | nr | CIPAC MT 47.1 and CIPAC MT 47.2 outline a procedure for the determination of persistent foam created when formulations are added to water before use. If this procedure is applied to the residual solution of SmartFresh™ ProPack, i.e. if the solution is filled in a glass measuring cylinder, the cylinder is closed with a glass stopper and subsequently inverted, it cannot not be excluded that the amount of CO2 still being released from the residual solution is able to generate enough pressure in the closed system to either cause the stopper to be forcibly expelled and/or cracking or shattering of the glass cylinder. This is a risk for the study director, which is why AgroFresh has requested an exemption for persistent foaming testing. However, this scenario is not applicable to real application conditions as the product is not applied in such a small closed system consisting of glassware. Instead the required amount of tablets for the treatment space (room, container, or trailer) is directly added to the SmartFresh™ ProPack, which is packed in single-use, stand-up pouches made of plastic. This stand-up pouch is not closed by any stoppers but open to allow for release of 1-MCP and CO2 into the treatment space Therefore, the product is used directly by adding the tablets SmartFresh Pro Tabs and SmartFresh Pro Activators in the smartFresh Pro Pack solution and the scenario described with the CIPAC method will not occur in real conditions. The safety of the Operator is not compromised by the liberation of 1-MCP and CO2 as there are precautions (all tablets are coated to delay the liberation of the 1-MCP) and directions for use (do not enter the room during application followed by ventilation before re-entry) in place to allow a safe application of the product. | N | Jacobson A. Study Number No. AF-15-108 | Acceptable The specific purpose for which persistence of foaming needs to be assessed is of less relevance within the context of this dossier, since the operator is supposed to leave the area as soon as possible after setting up the 1-MCP generator and is unlikely to contact any foam, if it is generated at all. |
| Suspensibility (KCP 2.8.3.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Spontaneity of dispersion (KCP 2.8.3.2) | FAO/WHO Manual, 2nd rev. of 1st ed., Nov. 2010 | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. RMJ8020C a.s. content: 2.0% w/w 1-MCP | The maximal disintegration time of AF-777 1-MCP Formulation pink was determined in 1 L of SmartFresh™ ProPack (Activator Solution). 1 tablet: 26 min 38 sec. 10 tablets: 32 min 14 sec. | Y | Kühne, M. (2011k) Report No. AF-11-024 | Acceptable |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | The maximal disintegration time of AF-777 1-MCP Formulation yellow was determined in 1 L of SmartFresh™ ProPack (Activator Solution). 1 tablet: 56 min 346 sec. 20 tablets: 59 min 32 sec. | Y | Kühne, M. (2011s) Report No. AF-11-035 | Acceptable |
|  |  | SmartFresh™ ProActivators Batch-No.711V3PVP | The maximum disintegration time of the test item are the following: Nb of tablets; Volume of ProPack; Maximum disintegration time (hours). 40; 1L; 4 hrs. 8; 200 mL; 2 hrs 30 min. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
| Dispersion stability (KCP 2.8.3.3) | nr | Nr | Not applicable for AF-777 | - | - | - |
| Degree of dissolution and dilution stability (KCP 2.8.4) | nr | Nr | Not applicable for AF-777 | - | - | - |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Wet sieve test (KCP 2.8.5.1.2) | nr | Nr | Not applicable for AF-777 | - | - | - |
| Dust content (KCP 2.8.5.2.1) | nr | Nr | Not applicable for AF-777 | - | - | - |
| Particle size of dust (KCP 2.8.5.2.2) | nr | Nr | Not applicable for AF-777 | - | - | - |
| Attrition (KCP 2.8.5.3) | FAO/WHO Manual, 2nd rev. of 1st ed., Nov. 2010 CIPAC MT 178.2 | SmartFreshTM ProTabs (AF-777 1-MCP Batch-No. RMJ8020C a.s. content: 2.0% w/w 1-MCP | The attrition of AF-777 1-MCP Formulation pink was determined to be 99.92% according to CIPAC method MT 178.2. CIPAC method MT 178.2 was used because it uses a “roller bank system” that AgroFresh did have available whereas CIPAC method MT 193 uses a “Friability Tester” which was not available at AgroFresh. Both methods are equivalent. In both tests the test material is sieved firstly in order to remove the fine particles. Then the tablets are mechanically stressed and sieved again. The percentage is calculated. MT 193: The tablets are put on the dish of the friability tester. The dish turns with 100 rotations (4 min). MT 178.2: The tablets are filled in the glass bottle of the roller bank system. The closed bottle is placed horizontally on the roller. The bottle carried out 4500 revolutions with a speed of 75 to 125 rounds per minutes. Comparing the stress conditions, MT178.2 is the worse-case test. | Y | Kühne, M. (2011j) Report No. AF-11-023 | Acceptable |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP | The attrition of AF-777 1-MCP Formulation yellow was determined to be 99.95% according to CIPAC method MT 178.2. | Y | Kühne, M. (2011r) Doc. No. AF-11-034 |  |
|  |  | SmartFresh™ ProActivators Batch-No.711V3PVP | The attrition of the test item was determined to be 100% according to CIPAC method MT 178.2. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 |  |
| Hardness and integrity (KCP 2.8.5.4) | nr | nr | Not applicable for AF-777 | - | - | - |
| Emulsifiability (KCP 2.8.6.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Emulsion stability (KCP 2.8.6.2) | nr | nr | Not applicable for AF-777 | - | - | - |
| Re-emulsifiability (KCP 2.8.6.3) | nr | nr | Not applicable for AF-777 | - | - | - |
| Flowability (KCP 2.8.7.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Pourability (KCP 2.8.7.2) | nr | nr | Not applicable for AF-777 | - | - | - |
| Dustability following accelerated storage (KCP 2.8.7.3) | nr | nr | Not applicable for AF-777 | - | - | - |
| Physical compatibility of tank mixes (KCP 2.9.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Chemical compatibility of tank mixes (KCP 2.9.2) | nr | nr | Not applicable for AF-777 | - | - | - |
| Adhesion to seeds (KCP 2.10.1) | nr | nr | Not applicable for AF-777 | - | - | - |
| Distribution to seed (KCP 2.10.2) | nr | nr | Not applicable for AF-777 | - | - | - |
| Other/special studies (KCP 2.11) | na | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:1F | Comparative release of 1-MCP from AF-777 1-MCP Formulation yellow and commercially available 1-MCP formulations: The release of one tablet of AF-777 1-MCP Formulation yellow (lowest application rate, containing 84 mg 1-MCP) was compared to 1-MCP release of SmartFresh™ 3.3%, EthylBloc® 0.14% and SmartFresh™ SmartTabs 0.63% (quantity was chosen to contain 84 mg of 1-MCP respectively) at 4°C. Initial 1-MCP release from AF-777 1-MCP Formulation yellow (20 min after activation): 0.2% of total 1-MCP. Full 1-MCP release (3 hrs after activation): AF-777 1-MCP Formulation yellow: 93.9% SmartFresh™ 3.3%: 86.2% EthylBloc® 0.14%: 98.8% SmartFresh™ SmartTabs 0.63%: 84.6% 1-MCP residue in AF-777 Spent Activator Solution: 1.1 10-4 mg/L 1-MCP (0.3% of total 1-MCP) The release characteristics of AF-777 1-MCP Formulation yellow are comparable to commercially available 1-MCP formulations. The delayed release of 1-MCP from AF-777 1-MCP Formulation yellow after activation is sufficient time for applicators to exit the application area. The small amount of 1-MCP remaining in the AF-777 Spent Activator Solution minimizes waste disposal issues. Due to safety reasons (build up of excess CO2 in laboratory test chamber potentially creating an implosion hazard), the release studies cannot be performed in the laboratory using the maximum number of tablets. However, it has been justified that the release rate for the minimum number of yellow tablets (1) will be proportional to the maximum number (20 or 25). That is, release of 1-MCP for both the minimum and maximum label use rate will exceed 90% by the end of the treatment. | Y | Verona, D. (2011a) Report No. AF-11-099 Jacobson A. (2015) AF-15-107 | Acceptable A theoretical statement has been submitted (Jacobson, 2015) that confirms that the amount of 1-MCP that will enter the headspace upon spending the tablets corresponds with the dosing information in the GAP. |
|  |  | SmartFreshTM ProTabs (AF-777 1-MCP formulation pink) Batch-No. RMJ8020C a.s. content: 2.0% w/w 1-MCP SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. RMJ8019A a.s. content: 2.0% w/w 1-MCP SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. FLW82:1F | Comparative release of 1-MCP from AF-777 1-MCP Formulation pink and AF-777 1-MCP Formulation yellow: The release of 5 tablets of AF-777 1-MCP Formulation pink (containing 84 mg 1-MCP) was compared to 1-MCP release of 1 tablet AF-777 1-MCP Formulation yellow (lowest application rate, containing 84 mg 1-MCP) at 4°C. 97.8% of 1-MCP in AF-777 1-MCP Formulation pink was released within 3 hrs. 1-MCP release of AF-777 1-MCP Formulation pink was equivalent to 104% of 1-MCP released from AF-777 1-MCP Formulation yellow. Thus, 1-MCP release characteristics of AF-777 1-MCP Formulation pink and AF-777 1-MCP Formulation yellow are comparable. | Y | Verona, D. (2011b) Report No. AF-11-132 |  |
| Other/special studies (KCP 2.11) | na | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. VEC294-2011 lot 2 a.s. content: 2.2% w/w 1-MCP SmartFreshTM ProTabs (AF-777 1-MCP formulation pink) Batch-No. VEC294-2011 lot 2 a.s. content: 2.1% w/w 1-MCP SmartFresh™ ProActivators Batch-No.711V3PVP | Tablet integrity: The tablet integrity was determined, on two separate pouches of AF-777 1-MCP (Yellow Tablets), the pouches also contained ProActivators Tablets. Pouch 1 contained 20 AF-777 Yellow tablets and 40 ProActivators, pouch 2 contained 1 AF-777 Yellow Tablet and 40 ProActivators. All tablets of both pouches were in good condition and no broken tablets were found. The tablet integrity was determined, on two separate pouches of AF-777 1-MCP (Pink Tablets), the pouches also contained ProActivators Tablets. Pouch 1 contained 25 AF-777 Pink tablets and 8 ProActivators, pouch 2 contained 1 AF-777 Pink Tablet and 8 ProActivators. All tablets of both pouches were in good condition and no broken tablets were found. | Y | Woolley, A.J; O’Connor, B.J Report No.41105350 | Acceptable |
| Other/special studies (KCP 2.11) | na | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. YY00BAK3A2 YY00D1L021 SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. YY00BAJ3A5 SmartFresh™ ProActivators (AF-777 1-MCP Activator Tablets) Batch-No. YY00CBE187 YY00D7P023 YY00C3F383 SmartFresh™ ProPack (AF-777 Activator Solution) Batch-No. YY00D16273 | Release of 1-Methylcyclopropene (1-MCP) from 2 Year Aged SmartFreshTM ProTabs (Yellow and Pink Formulations) with 2 Year Aged SmartFresh™ ProActivators and SmartFresh™ ProPack: The percent of 1-MCP released from the 2 year warehouse aged tablet was equivalent to 96% and 92% of the theoretically calculated 1-MCP in the tablet for the yellow and pink tablets, respectively. This demonstrates that 2 year old AF-777 tablets in conjunction with 2 year old activator tabs and activator solution released an amount equivalent to the amount of 1-MCP released in previous studies (AF-11-099 and AF-11-132) where only the yellow and pink AF-777 and activator tabs were aged for 2 years. In both cases, all of the 1-MCP is essentially released from the AF-777. | Y | Kurilla, K. (2015b) Report No. AF-15-102 | Acceptable The release rate of 1-MCP from the tablets was above 90% after storage for two years in the plastic foil pouch. |
| Other/special studies (KCP 2.11) | na | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. YY00D2R026 SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. YY00D1L022 | Assessment of the Physical Integrity of 2 year Aged SmartFreshTM ProTabs (Pink and Yellow Tablets) upon opening of the pouch: The colour and the coating surface of the SmartFreshTM ProTabs was unchanged upon opening the pouch following more than 2 years storage. There was no weight loss on testing in the friabilator. | N | Wehmeyer, F. (2016) Report No. AF-16-016 |  |
| Other/special studies (KCP 2.11) | na | SmartFreshTM ProTabs (AF-777 1-MCP Formulation yellow) Batch-No. A290EBO015 a.s. content: 2.11% w/w 1-MCP | Determination of the 1-MCP active substance, 1-CMP and 3-CMP impurities concentration in AF-777 yellow tablets: The analysis of 1-MCP in AF-777 yellow tablets has demonstrated that the content (2.11%) meets the certified limit range (1.5 -2.5 wt.%) for the active. The analysis of the impurities, 1-CMP and 3-CMP has demonstrated that they were not detectable with a LOD < 0.015%. The analyses demonstrated that the composition of the batch meet the current product specifications for the active substance and demonstrate that the batch meet the certified limits set for the impurities. | Y (the confirmation of the 3-CMP by GC/MS was not performed under GLP) | Kurilla, K. (2016a) Report No. AF-15-110 Baldi, BG. (2019) Report No. AF-19-016 | Acceptable The content of the impurities before storage is determined to be below the limit of detection of 0.05%. Both impurities are process impurities and their formation on storage is not expected. The content of 1-MCP was determined using the validated GC/FID method. The CIPAC method (CPAC/4667/m) was used for the determination of the impurities in the formulation. It is noted that the CIPAC method for the determination of the impurities was not GLP compliant. However, it is stated by the applicant that laboratories in the USA are not certified by any governmental agency, but are subject to official inspections. |
| Other/special studies (KCP 2.11) | na | SmartFreshTM ProTabs (AF-777 1-MCP Formulation pink) Batch-No. YY00FAF012 a.s. content: 2.01% w/w 1-MCP | Determination of the 1-MCP active substance, 1-CMP and 3-CMP impurities concentration in AF-777 pink tablets: The analysis of 1-MCP in AF-777 pink tablets has demonstrated that the content (2.01%) meets the certified limit range (1.5 -2.5 wt.%) for the active. The analysis of the impurities, 1-CMP and 3-CMP has demonstrated that they were not detectable with a LOD < 0.015% and thus, the content of each of these two impurities is well below the EU specifications fixed at < 0.02%. The analyses demonstrated that the composition of the batch meet the current product specifications for the active substance and demonstrate that the batch meet the certified limits set for the impurities | Y (the confirmation of the 3-CMP by GC/MS was not performed under GLP) | Kurilla, K. (2016b) Report No. AF-16-001 Baldi, BG. (2019) Report No. AF-19-016 |  |
| Other/special studies (KCP 2.11) | Corrosiveness | The discrepancy between Regulation (EC) No 1107/2009 and CLP regulation was discussed at EFSA in the Peer Review meeting 150 (see: Outcome of the pesticides peer review meeting on general recurring issues in physical and chemical properties and analytical methods, approved 16 April 2019, EFSA Supporting publication 2019:EN-1623). The meeting had agreed that a full set of data for classification and labelling with regard to physical and chemical properties could not be requested at the moment. It should be included in an amendment of the data requirements Regulation in alignment with CLP Regulation. Thus, the harmonised methods need to be included in the next revision of the method Commission communication (2013/C 95/01 and 2013/C 95/02). Based on the exact composition of the solution, it is shown that no study is necessary, as the H290 classified ingredient (sodium hydroxide) is only corrosive under strongly basic conditions which are not given in the acidic mixture. Additionally, there is a neutralization reaction with the present excess of citric acid, the products of which are not classified. A position paper is included for further details (SCC (2022), Report No. PP278-00047). |  |  |  | Acceptable The pH of SmartFresh™ ProPack (AF-777 Activator Solution) is very low. In addition, H290 classified component presents in the formulation. Therefore, a test was requested to confirm that no classification of the solution as may be corrosive to metals (H290) is required in accordance with the CLP Regulation ((EC) No 1272/2008. The applicant presented a position paper demonstrating that no classification is required on the basis of the composition of the solution. The statement has been found acceptable. |
| na = not applicable, nr = not relevant |  |  |  |  |  |  |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

| Comments of zRMS: | The integrity of the packaging material for SmartFreshTM ProTabs has been demonstrated in the accelerated storage stability study as well as the shelf-life study at ambient temperature. The package integrity for the SmartFresh™ ProActivators pouch and SmartFresh™ ProPack stand up pouch have been demonstrated in the accelerated storage stability (Report AF-15-109). In conclusion, a provisional 2-year shelf-life is granted in commercial packaging. A post-authorization data gap has been assigned for: -post long-term (ambient) storage pH for the ProPacks -post long-term (ambient) data on packaging integrity of the ProActivators and ProPacks. |
| --- | --- |

Table 4.1-7:	Description of the SmartFreshTM ProTabs pouch

| Type | Description |
| --- | --- |
| Description: | Flat, 3 sided seal poly/foil laminate white pouch |
| Type of material: | PET/White PE/Foil/PE (i.e., PolyEthylene Terephthalate / PolyEthylene / Aluminium / PolyEthylene) |
| Dimensions: | 2 sizes: - 20 cm * 11,5 cm - 25,5 cm *20,5 cm |

Tablet weight:

- SmartFresh™ ProTabs Yellow: 4.2 g

- SmartFresh™ ProTabs Pink: 0.84 g

Table 4.1-8:	Description of the SmartFresh™ ProActivators pouch

| Type | Description |
| --- | --- |
| Description: | Flat, 3 sided seal poly/foil laminate white pouch |
| Type of material: | PET/White PE/Foil/PE (i.e., PolyEthylene Terephthalate / PolyEthylene / Aluminium / PolyEthylene) |
| Dimensions: | 2 sizes: - 20 cm * 11,5 cm - 25,5 cm *20,5 cm |

Tablet weight:  SmartFresh™ ProActivators: 1.6 g

Table 4.1-9:	Description of the SmartFresh™ ProPack stand up pouch

| Type | Description |
| --- | --- |
| Description: | Laminated bag with bottom gusset |
| Type of material: | PETC/BON/LLDPE (i.e., Crystallized PolyEthylene Terephthalate / Biaxially Oriented Nylon / Linear Low Density Polyethylene) |
| Dimensions: | 4 sizes: - 130*230*80 mm (W*L*G) - 203x290x100 mm (W*L*G) - 235x370x100 mm (W*L*G) - 300*450*160 mm (W*L*G) |

The resistance of the packaging material to its contents has been tested according to EPA/OPPTS 830.6313 and 830.6320 (Verona, D. (2011e, f)) and found to be acceptable, please refer to KCP 2.7.1. No change in appearance was observed after storage. There was no observable deterioration in the surfaces of the pouches, and the integrity of the pouch seals was maintained.

The Spent ProPackTM Solution and the packaging should be disposed according to national waste disposal regulations.

Protective clothing should be either discarded or washed with soap and water.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Annex point | Author | Year | Title Source (where different from company) Company, Report No. GLP or GEP status (where relevant) Published or Unpublished | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.7.5/03 | Kurilla, K. | 2014a | Storage Stability and Corrosion Characteristics of AF-777 1-Methylcyclopropene Formulation for 24 months at 25°C (Pink) Formulation (Vector Pink VEC294-2011 Lot 2) Report No. AF-14-008 GLP, unpublished | N | AgroFresh |
| KCP 2.4.2 ; KCP 2.11 | SCC | 2022 | SmartFresh™ ProPack Activator Solution: Influence of a H290 classified component on the classification of the mixture Report No. PP278-00047 Non GLP, unpublished | N | AgroFresh |
| KCP 2.7.1/01; KCP 2.7.5/01 | Wehmeyer F. | 2016 | GLP Accelerated Storage Stability of SmartFresh ProActivators and SmartFresh ProPack: Dissolution, Attrition Resistance, pH and Acidity Report No. AF-15-109 GLP, unpublished | N | AgroFresh |
| KCP 2.7.1/02; KCP 2.7.5/02 | Wehmeyer F. | 2017 | GLP Accelerated Storage Stability of SmartFresh ProTabs Yellow: pH, Desintegration and Attrition Resistance Report No. AF-17-056 GLP, unpublished | N | AgroFresh |
| KCP 2.7.1/03; KCP 2.7.5/03 | Wehmeyer F. | 2017 | GLP Accelerated Storage Stability of SmartFresh ProTabs Pink: pH, Desintegration and Attrition Resistance Report No. AF-17-057 GLP, unpublished | N | AgroFresh |
| KCP 2.7.5/04 | Kurilla, K. | 2014b | Storage Stability and Corrosion Characteristics of AF-777 1-Methylcyclopropene Formulation for 24 months at 25 °C (Yellow) Formulation (Vector Yellow VEC298-2011 Lot 1) Report No. AF-14-006 GLP, unpublished | N | AgroFresh |
| KCP2.7.1/05 | Wehmeyer F. | 2022 | GLP determination of the pH of 1% aqueous SmartFresh ProActivators tablet after storage for two weeks at 54°C Report No. AF-22-007 GLP, unpublished | N | AgroFresh |
| KCP 2.8.2/01 | Jacobson A. | 2015 | Position paper: Rationale for not performing a persistent foam study with a Spent ProPack Solution Report No. AF-15-108 GLP, unpublished | N | AgroFresh |
| KCP 2.11/07 | Kurilla, K. | 2015 | Release of 1-Methylcyclopropene (1-MCP) from 2 Year Aged SmartFresh™ ProTabs (Yellow and Pink Formulations) with 2 Year Aged SmartFresh™ ProActivators and SmartFresh™ ProPack Report No. AF-15-102 GLP, unpublished | N | AgroFresh |
| KCP 2.11/08 | Wehmeyer, F. | 2016 | Assessment of the Physical Integrity of 2 year Aged SmartFresh™ ProTabs (Pink and Yellow Tablets) upon opening of the pouch. Report No.: AF-16-016 GLP, Unpublished | N | AgroFresh |
| KCP 2.11/09 | Kurilla, K. | 2016a | Analysis of SmartFreshTM ProTabs (Yellow) Lot A290EBO015 for 1-Methylcyclopropene (1-MCP), 1-chloro-2-methylpropene (1-CMP) and 3-chloro-2-methylpropene (3-CMP). AgroFresh Inc., Collegeville, USA Report No. AF-15-110 GLP, unpublished | N | AgroFresh |
| KCP 2.11/10 | Kurilla, K. | 2016b | Analysis of SmartFreshTM ProTabs (Pink) Lot YY00FAF012 for 1-Methylcyclopropene (1-MCP), 1-chloro-2-methylpropene (1-CMP) and 3-chloro-2-methylpropene (3-CMP). AgroFresh Inc., Collegeville, USA Report No. AF-16-001 GLP, unpublished | N | AgroFresh |
| KCP 2.11/11 | Baldi BG. | 2019 | Levels of 1-chloro-2-methyl-propene (1-CMP) and 3-chloro-2-methylpropene (3-CMP) impurities of 1-methylcyclopropene (1-MCP) Determined in SmartFreshTM ProTabs AgroFresh 3 Springhouse innovation park, suite 100, lower Gwynedd, PA 19002, USA Report No. AF-19-016 Non GLP, unpublished | N | AgroFresh |
| KCP 2.11/08 | Jacobson A. | 2015 | Position paper : Analysis demonstrating the release rate of 1-MCP from SmartFresh ProTabs is proportional for the minimum and maximum label rate. Report No. AF-15-107 Non GLP, unpublished | N | AgroFresh |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

The studies listed below are not submitted herewith as these reports have been either evaluated during the EU review of 1-MCP or during national registration of AF-777.

| Annex point | Author | Year | Title Source (where different from company) Company, Report No. GLP or GEP status (where relevant) Published or Unpublished | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP 2.1/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Pink): Determination of the Physico-Chemical Property “Appearance (Color, Physical State and Odor)” Harlan Laboratories Ltd. Itingen Report No.: AF-11-021 D15878 GLP, unpublished | N | AgroFresh |
| KCP 2.1/02 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Determination of the Physico-Chemical Property “Appearance (Color, Physical State and Odor)” Harlan Laboratories Ltd. Itingen Report No.: AF-11-028 D15766 GLP, unpublished | N | AgroFresh |
| KCP 2.1/03 | Kühne, M. | 2011 | AF-777 Activator Solution: Determination of the Physico-Chemical Property “Appearance (Color, Physical State and Odor)” Harlan Laboratories Ltd. Itingen Report No.: AF-11-047 D16036 GLP, unpublished | N | AgroFresh |
| KCP 2.1/04 | Woolley A.J O’Connor B.J | 2012 | AF-777 Activator Tablet: Determination of physico-chemical properties (including the tablet integrity for AF-777 1-methylcyclopropene yellow and pink) Harlan Laboratories Ltd UK Report No. 41105350 GLP, unpublished | N | AgroFresh |
| KCP 2.1/05 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: Determination of the Physico-Chemical Property “Appearance (Color, Physical State and Odor)” Harlan Laboratories Ltd. Itingen Report No.: AF-11-011 D16148 GLP, unpublished | N | AgroFresh |
| KCP 2.2.1/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene (Yellow): Determination of the Explosive Properties Harlan Laboratories Ltd. Itingen Report No.: AF-11-068 GLP, unpublished | N | AgroFresh |
| KCP 2.2.2/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Determination of Oxidation / Reduction (Chemical Incompatibility) Harlan Laboratories Ltd. Itingen Report No.: AF-11-029 D15777 GLP, unpublished | N | AgroFresh |
| KCP 2.2.2/02 | Kühne, M. | 2011 | AF-777 Activator Solution: Determination of Oxidation / Reduction (Chemical Incompatibility) Harlan Laboratories Ltd. Itingen Report No.: AF-11-048 D16047 GLP, unpublished | N | AgroFresh |
| KCP 2.2.2/03 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.2.2/04 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: Determination of Oxidation / Reduction (Chemical Incompatibility) Harlan Laboratories Ltd. Itingen Report No.: AF-11-012 D16150 GLP, unpublished | N | AgroFresh |
| KCP 2.3.1/01 | Kühne, M. | 2011 | AF-777 Activator Solution: Determination of the Physico-Chemical Property “Flash Point” Harlan Laboratories Ltd. Itingen Report No.: AF-11-049 D16060 GLP, unpublished | N | AgroFresh |
| KCP 2.3.1/02 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: Determination of the Physico-Chemical Property “Flash Point” Harlan Laboratories Ltd. Itingen Report No.: AF-11-013 D16172 GLP, unpublished | N | AgroFresh |
| KCP 2.3.2/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Determination of the Physico-Chemical Property “Flammability” Harlan Laboratories Ltd. Itingen Report No.: AF-11-030 D15790 GLP, unpublished | N | AgroFresh |
| KCP 2.3.2/02 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.3.3/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Determination of the Physico-Chemical Property “Relative Self-Ignition Temperature” Harlan Laboratories Ltd. Itingen Report No.: AF-11-031 D15801 GLP, unpublished | N | AgroFresh |
| KCP 2.3.3/02 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.3.3/03 | Atwal, S.S. et al. | 2011 | AF-777 Activator Solution: Determination of Auto-Ignition Temperature (liquids and gases) Harlan Laboratories Ltd. Shardlow Report No.: AF-11-046 D26105 41100834 GLP, unpublished | N | AgroFresh |
| KCP 2.3.3/04 | Atwal, S.S. et al. | 2011 | AF-777 Spent Activator Solution: Determination of Auto-Ignition Temperature (liquids and gases) Harlan Laboratories Ltd. Shardlow Report No.: AF-11-045 D16183 41100267 GLP, unpublished | N | AgroFresh |
| KCP 2.4.1/01 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.4.1/02 | Kühne, M. | 2011 | AF-777 Activator Solution: pH-Determination and the Determination of Acidity of Alkalinity Harlan Laboratories Ltd. Itingen Report No.: AF-11-050 D16093 GLP, unpublished | N | AgroFresh |
| KCP 2.4.2/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): pH-Determination and the Determination of Acidity or Alkalinity Harlan Laboratories Ltd. Itingen Report No.: AF-11-032 D15823 GLP, unpublished | N | AgroFresh |
| KCP 2.4.2/02 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.4.2/03 | Kühne, M. | 2011 | Refer to KCP 2.4.1/03 | N | AgroFresh |
| KCP 2.4.2/04 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: pH-Determination and Determination of Acidity of Alkalinity Harlan Laboratories Ltd. Itingen Report No.: AF-11-014 D16205 GLP, unpublished | N | AgroFresh |
| KCP 2.5.1/01 | Kühne, M. | 2011 | AF-777 Activator Solution: Determination of the Physico-Chemical Property “Viscosity” Harlan Laboratories Ltd. Itingen Report No.: AF-11-051 D16115 GLP, unpublished | N | AgroFresh |
| KCP 2.5.1/02 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: Determination of the Physico-Chemical Property “Viscosity” Harlan Laboratories Ltd. Itingen Report No.: AF-11-015 D16216 GLP, unpublished | N | AgroFresh |
| KCP 2.5.2/01 | Kühne, M. | 2011 | AF-777 Activator Solution: Determination of the Physico-Chemical Property “Surface Tension of an Aqueous Solution” Harlan Laboratories Ltd. Itingen Report No.: AF-11-052 D16126 GLP, unpublished | N | AgroFresh |
| KCP 2.5.2/02 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: Determination of the Physico-Chemical Property “Surface Tension of an Aqueous Solution” Harlan Laboratories Ltd. Itingen Report No.: AF-11-016 D16227 GLP, unpublished | N | AgroFresh |
| KCP 2.6.1/01 | Kühne, M. | 2011 | AF-777 Activator Solution: Determination of the Physico-Chemical Property “Relative Density” Harlan Laboratories Ltd. Itingen Report No.: AF-11-053 D16104 GLP, unpublished | N | AgroFresh |
| KCP 2.6.1/02 | Kühne, M. | 2011 | AF-777 Spent Activator Solution: Determination of the Physico-Chemical Property “Relative Density” Harlan Laboratories Ltd. Itingen Report No.: AF-11-017 D16238 GLP, unpublished | N | AgroFresh |
| KCP 2.6.2/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Pink): Determination of the Bulk Density Harlan Laboratories Ltd. Itingen Report No.: AF-11-022 D15880 GLP, unpublished | N | AgroFresh |
| KCP 2.6.2/02 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Determination of the Bulk Density Harlan Laboratories Ltd. Itingen Report No.: AF-11-033 D15834 GLP, unpublished | N | AgroFresh |
| KCP 2.6.2/03 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.7.1/01 | Verona, D. | 2011 | Accelerated Storage Stability of AF-777 1-Methylcyclopropene Formulation AgroFresh Inc., Spring House Report No.: AF-11-102 AF-P-11-19 GLP, unpublished | N | AgroFresh |
| KCP 2.7.1/02 | Verona, D. | 2011 | Comparative Release of AF-777 1-Methylcyclopropene Formulation Packaged with AF 777 Activator Tablets Before and After Accelerated Storage Conditions AgroFresh Inc., Spring House Report No.: AF-11-134 AF-P-11-26 GLP, unpublished | N | AgroFresh |
| KCP 2.7.2/01 | Kurilla, K. | 2013 | INTERIM REPORT: Storage Stability and Corrosion Characteristics of AF-777 1-Methylcyclopropene (Yellow) Formulation AgroFresh Inc., Collegeville Report No.: AF-13-005 AF-P-11-32 GLP, unpublished | N | AgroFresh |
| KCP 2.7.2/02 | Kurilla, K. | 2013 | INTERIM REPORT: Storage Stability and Corrosion Characteristics of AF-777 1-Methylcyclopropene (Pink) Formulation AgroFresh Inc., Collegeville Report No.: AF-13-007 AF-P-11-35 GLP, unpublished | N | AgroFresh |
| KCP 2.7.4/01 | Kühne, M. | 2011 | AF-777 Activator Solution: Low Temperature Stability Harlan Laboratories Ltd. Itingen Report No.: AF-11-054 D16137 GLP, unpublished | N | AgroFresh |
| KCP 2.7.5/01 | Kurilla, K. | 2014 | Storage Stability and Corrosion Characteristics of AF-777 1-Methylcyclopropene Formulation for 24 months at 25°C (Yellow) Formulation AgroFresh Inc., Collegeville Report No.: AF-14-006 AF-P-11-32 GLP, unpublished | N | AgroFresh |
| KCP 2.7.5/02 | Kurilla, K. | 2014 | Storage Stability and Corrosion Characteristics of AF-777 1-Methylcyclopropene Formulation for 24 months at 25°C (Pink) Formulation AgroFresh Inc., Collegeville Report No.: AF-14-008 AF-P-11-35 GLP, unpublished | N | AgroFresh |
| KCP 2.8.3.2/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Pink): Disintegration Time and Degree of Dispersion or Dissolution Harlan Laboratories Ltd. Itingen Report No.: AF-11-024 D15913 GLP, unpublished | N | AgroFresh |
| KCP 2.8.3.2/02 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Disintegration Time and Degree of Dispersion or Dissolution Harlan Laboratories Ltd. Itingen Report No.: AF-11-035 D15867 GLP, unpublished | N | AgroFresh |
| KCP 2.8.3.2/03 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.8.5.3/01 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Pink): Degree of Attrition Harlan Laboratories Ltd. Itingen Report No.: AF-11-023 D15902 GLP, unpublished | N | AgroFresh |
| KCP 2.8.5.3/02 | Kühne, M. | 2011 | AF-777 1-Methylcyclopropene Formulation (Yellow): Degree of Attrition Harlan Laboratories Ltd. Itingen Report No.: AF-11-034 D15856 GLP, unpublished | N | AgroFresh |
| KCP 2.8.5.3/03 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.11/01 | Verona, D. | 2011 | Refer to Doc K – Part C | N | AgroFresh |
| KCP 2.11/02 | Verona, D. | 2011 | Refer to Doc K – Part C | N | AgroFresh |
| KCP 2.11/03 | Woolley A.J O’Connor B.J | 2012 | Refer to KCP 2.1/04 | N | AgroFresh |
| KCP 2.11/04 | Kurilla, K. | 2015 | Release of 1-Methylcyclopropene (1-MCP) from 2 Year Aged SmartFresh™ ProTabs (Yellow and Pink Formulations) with 2 Year Aged SmartFresh™ ProTabs Activator Tablets and Activator Solutions. Report No.: AF-15-045 AF-P-15-09 GLP, Unpublished | N | AgroFresh |
| KCP 2.11/05 | Kurilla, K. | 2015 | Release of 1-Methylcyclopropene (1-MCP) from 2 Year Aged SmartFresh™ ProTabs (Yellow and Pink Formulations) with 2 Year Aged SmartFresh™ ProActivators and SmartFresh™ ProPack. Report No.: AF-15-102 AF-P-15-09 GLP, Unpublished | N | AgroFresh |

List of data submitted by the applicant and not relied on

None

List of data relied on and not submitted by the applicant but necessary for evaluation

None

- Additional data on the physical, chemical and technical properties of the active substance

  - 1-Methylcyclopropene

No additional data with 1-MCP are necessary nor available.

  - Active substance 2

The product AF-777 contains only 1 active substance.