REGISTRATION REPORT Part B Section 1: Identity Section 2: Physical and chemical properties Section 4: Further information Detailed summary of the risk assessment

Product code: A17960B Product name(s): Fortenza Chemical active substance(s): Cyantraniliprole, 600 g/L

Interzonal Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (New Authorization)

Applicant: Syngenta Submission date: 30/10/2020 MS Finalisation date: August 2021 (initial Core Assessment) December 2021 (final Core Assessment)

Version history

| When | What |
| --- | --- |
| 10/2020 | Initial dRR - Syngenta |
| 08/2021 | Initial zRMS assessment The report in the dRR format has been prepared by the Applicant, therefore all comments, additional evaluations and conclusions of the zRMS are presented in grey commenting boxes. Minor changes are introduced directly in the text and highlighted in grey. Not agreed or not relevant information are struck through and shaded for transparency. |
| December 2021 | Final report (Core Assessment updated following the commenting period) No additional information or assessments after the commenting period. |

Table of Contents

1	Section 1: Identity of the plant protection product	4

1.1	Applicant (KCP 1.1)	4

1.2	Producer of the plant protection product and of the active substances (KCP 1.2)	4

1.2.1	Producer(s) of the preparation	4

1.2.2	Producer(s) of the active substance(s)	4

1.2.3	Statement of purity (and detailed information on impurities) of the active substance(s)	4

1.2.3.1	Cyantraniliprole	4

1.3	Trade names and producer’s development code numbers for the preparation (KCP 1.3)	5

1.4	Detailed quantitative and qualitative information on the composition of the preparation (KCP 1.4)	5

1.4.1	Composition of the plant protection product (KCP 1.4.1)	5

1.4.2	Information on the active substance(s) (KCP 1.4.2)	5

1.4.3	Information on safeners, synergists and co-formulants (KCP 1.4.3)	5

1.5	Type and code of the plant protection product (KCP 1.5)	5

1.6	Function (KCP 1.6)	6

2	Section 2: Physical, chemical and technical properties of the plant protection product	7

3	Section 3 is presented as a separate document	15

4	Section 4: Further information on the plant protection product	16

4.1	Packaging and Compatibility with the Preparation (KCP 4.4)	16

Appendix 1	Lists of data considered in support of the evaluation	18

Appendix 2	Additional data on the physical, chemical and technical properties of the active substance	21

Appendix 3:	Storage stability data before and after storage for two weeks at 54°C in HDPE packaging	22

Appendix 4:	Storage stability data before and after storage for two years at 25°C in HDPE packaging	24

zRMS comments: All studies except the three years storage stability study at 25oC have been already evaluated and accepted by the zRMS in the course of the zonal evaluation of Fortenza performed for purposes of authorisation as a seed treatment in oilseed rape and finalised in July 2020. Although no authorisation for uses in oilseed rape could be granted, zRMS evaluation of the study remain valid and presented below conclusions were taken from the Core Assessment, Part B, Section 1,2,4 of July 2020, in the acceptance of the study results, the new proposed intended uses of the plant protection product were taken into account in accordance with the data contained in the current Part B Section 0.

Sufficient data on identity, physical and chemical properties and other information are available for the plant protection product and the contained technical active substance.

Data gap: none

# Section 1: Identity of the plant protection product

## Applicant (KCP 1.1)

Name:	SYNGENTA CROP PROTECTION AG

Contact:	xxx

Syngenta Crop Protection AG

Rosentalstrasse 67

B5.3.150-14

CH-4058 Basel

Switzerland

Telephone nr.:	xxx

Fax number:	xxx

E-mail: 	xxx

## Producer of the plant protection product and of the active substances (KCP 1.2)

### Producer(s) of the preparation

Confidential information or data are provided separately (Part C).

### Producer(s) of the active substance(s)

Confidential information or data are provided separately (Part C).

### Statement of purity (and detailed information on impurities) of the active
substance(s)

#### Cyantraniliprole

| Test Substance | EU agreed minimum purity (COMMISSION DIRECTIVE 2016/1414) |
| --- | --- |
| Cyantraniliprole | 940 g/kg |
| Relevant impurity | EU agreed limits |
| IN-Q6S09 | < 1 mg/kg |
| IN-RYA13 | < 20 mg/kg |
| Methanesulfonic acid | < 2 g/kg |
| Acetonitrile | < 2 g/kg |
| Heptane | < 7 g/kg |
| 3-picoline | < 3 g/kg |

Pure cyantraniliprole in A17960B

| content of pure active substance: | 600.0 g/L | 47.8 % w/w1 |
| --- | --- | --- |
| limits : | 575.0 – 625.0 g/L | 45.8 - 49.8 % w/w |

1 Based on the density of the formulation of 1255 g/L

Technical cyantraniliprole in A17960B

| at a minimum purity of the technical active substance of 94 % w/w. |  |  |
| --- | --- | --- |
| content of technical active substance: | 638.3 g/L | 50.9 % w/w1 |
| limits : | 613.3 – 663.3 g/L | 48.9 - 52.9 % w/w |

1 Based on the density of the formulation of 1255 g/L

An assessment of equivalence is not required since sources for the active substance have been approved previously.

## Trade names and producer’s development code numbers for the preparation (KCP 1.3)

| Trade name: | Please refer to Registration Report Part A for the relevant country (or) |
| --- | --- |
| Trade name: | Fortenza |
| Company code number: | A17960B |

## Detailed quantitative and qualitative information on the composition of the
preparation (KCP 1.4)

### Composition of the plant protection product (KCP 1.4.1)

The product A17960B was not evaluated previously as a representative formulation (same uses and same GAPs) during the EU review of the active substances Cyantraniliprole.

The content of Cyantraniliprole in A17960B is given under point 1.2.3

The maximum amount of relevant impurities has been addressed in point 1.2.3

Information on the variants is addressed under point 1.4.2

Information on the formulants including safeners and synergists is confidential and is included in Part C (Confidential information).

### Information on the active substance(s) (KCP 1.4.2)

Table 1.4.2-1:	Information on Cyantraniliprole

| Type | Name/Code Number |  |
| --- | --- | --- |
| ISO common name | Cyantraniliprole | Variant: not relevant |
| CAS No. | 736994-63-1 |  |
| EC No. | not available |  |
| CIPAC No. | not available |  |

### Information on safeners, synergists and co-formulants (KCP 1.4.3)

CONFIDENTIAL information is provided separately (Part C).

## Type and code of the plant protection product (KCP 1.5)

| Type: Flowable concentrate for seed treatment | [Code: FS] |
| --- | --- |

## Function (KCP 1.6)

Insecticide.

# Section 2: Physical, chemical and technical properties of the plant protection product

All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable.  The appearance of the product is that of cream coloured liquid, with a chalky odour. It is not explosive, has no oxidising properties. The product is not flammable, with no flash point detected below 101 °C. It has a self ignition temperature of 450 °C. In aqueous solution, it has a pH value around 5.3. There is no effect of low and high temperature on the stability of the formulation, since after 7 days at 0 °C and 14 days at 54 °C, neither the active ingredient content nor the technical properties were changed. The formulation has been additionally stored for 2Y at 25ºC and for 3Y at 25°C. The stability data indicate a shelf life of at least 3 years at ambient temperature when stored in HDPE.  Its technical characteristics are acceptable for a FS formulation.

The intended concentration of use is 37.5% to 100%.

Justified Proposals for Classification and Labelling (KCP 12) for physical chemical part only

According to Regulation (EC) No. 1272/2008 no specific labelling or classification is proposed based on the measured physico-chemical properties of A17960B

Notifier Proposals for Risk and Safety Phrases (KCP 12)

According to Regulation (EC) No. 1272/2008 no specific labelling or classification is proposed based on the measured physico-chemical properties of product A17960B

Compliance with FAO specifications:

There is no FAO specification for A17960B.

Formulation used for tests

A17960B (and variant A17960A).

Table 2-1:	Physical, chemical and technical properties of the plant protection product

All tests have been performed under GLP, except where mentioned.

All tests were conducted using material from batches:

SMU3HL016 containing 619 g/L of cyantraniliprole. (Abraham N., 2014, Syngenta file N° A17960B_10050)

Few safety studies were carried out using variant A17960A which has practically identical chemical composition to A17960B.  An assessment of the similarity of the two formulation compositions is provided in Registration Report, Part C.  It is therefore proposed to rely on the safety data from A17960A to support A17960B. Tests carried out on A17960A, used material from batch SMU1AP002 containing 609 g/L of cyantraniliprole. (Das R., 2011, Syngenta file N° A17960B_10014).

| Annex point | Method used / deviations | Test material | Findings | GLP Y/N | Reference | Acceptability / comments |
| --- | --- | --- | --- | --- | --- | --- |
| Colour and physical state (KCP 2.1) | Visual and organoleptic test | 619 g/L cyantraniliprole FS (A17960B) | Colour: cream coloured Physical state: liquid Odour: chalky | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Explosive properties (KCP 2.2.1) | EEC A.14 | 609 g/L cyantraniliprole FS (A17960A) | Not an explosive substance. | Y | Jackson W.A., 2011 A17960A_10015; VV-396281 | Accepted. |
| Oxidizing properties (KCP 2.2.2) | EEC A.21 | 609 g/L cyantraniliprole FS (A17960A) | Not an oxidising substance. | Y | Jackson W.A., 2011 A17960A_10015; VV-396281 | Accepted. |
| Flash point (KCP 2.3.1) | EEC A.9 | 619 g/L cyantraniliprole FS (A17960B) | Not detected below 101°C | Y | Jackson W.A., 2011 A17960B_10008; VV-396538 | Accepted. |
| Flammability (KCP 2.3.2) | - | - | Not classified in terms of its flash point | - | - | - |
| Self-heating (KCP 2.3.3) | EEC A.15 | 609 g/L cyantraniliprole FS (A17960A) | 450°C ± 5 °C | Y | Jackson W.A., 2011 A17960A_10015; VV-396281 | Accepted. |
| Acidity or alkalinity and pH (KCP 2.4.1) | CIPAC MT 191 | 619 g/L cyantraniliprole FS (A17960B) | 0.07 % (calculated as H2SO4) |  | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| pH of a 1% aqueous dilution, emulsion or dispersion (KCP 2.4.2) | CIPAC MT 75.3 | 619 g/L cyantraniliprole FS (A17960B) | pH of a 1 % dispersion in water: 5.3 (at 25oC) pH undiluted: 4.3 (at 25oC) |  | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Viscosity (KCP 2.5.1) | CIPAC MT 192 | 619 g/L cyantraniliprole FS (A17960B) | At 20°C:- 100 s-1: 115 mPa.s 20 s-1: 304 mPa.s At 40°C:- 100 s-1: 88 mPa.s 20 s-1: 239 mPa s | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Surface tension (KCP 2.5.2) | EEC A.5 | 619 g/L cyantraniliprole FS (A17960B) | 6% in water at 20°C: 28.3 mN/m 1.5% in water at 20°C: 30.5 mN/m 0.1% in water at 20°C: 34.3 mN/m Undiluted at 20°C: 32.7 mN/m | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Density (KCP 2.6.1) | OED 109 | 619 g/L cyantraniliprole FS (A17960B) | At 20°C: 1.255 g/cm3 | Y | Abraham N., 2014, A17960B_10050; VV-406757 | Accepted. |
| Bulk density (KCP 2.6.2) | - | - | Not relevant (liquid) | - | - | - |
| Storage Stability after 14 days at 54º C (KCP 2.7.1) | CIPAC MT 46.3 The active ingredient content in the formulation was analysed following the method ST-18/l. HPLC method with external standard calibration. For full validation details please refer to the Section 5. | 619 g/L cyantraniliprole FS (A17960B) | The formulation is physically and chemically stable after storage for two weeks at 54°C in the following packaging material: - High density polyethylene pack (HDPE) For full details, please see Appendix 3 | N | Kundel P., 2014 A17960B_10051; VV-406758 | The product showed no significant physical changes after accelerated storage and all performance properties were within acceptable limits. No toxicologically, ecotoxicologically or environmentally relevant impurities are formed upon storage, evaluation of this parameter after storage is not necessary. |
| Stability after storage for other periods and/or temperatures (KCP 2.7.2) | - | - | Not relevant as the formulation is stable at 54°C | - | - | - |
| Minimum content after heat stability testing (KCP 2.7.3) | - | - | Not relevant as the formulation is stable at 54°C | - | - | - |
| Effect of low temperatures on stability (KCP 2.7.4) | CIPAC MT 39.3 CIPAC MT 185 CIPAC MT 184 | 619 g/L cyantraniliprole FS (A17960B) | After storage of 7 days at 0 °C no separation was observed. After allowing the test sample to reach room temperature over a period of 24 h and inverting once no separation was observed. Wet sieve test: a residue of 0.02 % was found on the test sieve Suspensibility (gravimetrical assay): Concentration 6% in CIPAC water D at 30°C: 99% Concentration 1.5% in CIPAC water D at 30°C: 97% | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted |
| Ambient temperature shelf life (KCP 2.7.5) | CIPAC MT 46.3 The active ingredient con-tent in the formulation was analysed fol-lowing the method ST-18/l. HPLC method with external standard calibration. For full validation details please refer to the Section 5. | 619 g/L cyantraniliprole FS (A17960B) | The formulation is physically and chemically stable after storage for two years at 25°C in the following packaging material: - High density polyethylene pack (HDPE) For full details, please see Appendix 4. | N | Kundel P., 2016 A17960B_10264; VV-463100 | The product showed no significant physical changes after storage for 2 and 3 years and all performance properties were within acceptable limits when storage in HDPE containers. Extrapolation from HDPE to any of the following HDPE co-extruded:- EVOH, fluorinated HDPE and polyamide is acceptable. No toxicologically, ecotoxicologically or environmentally relevant impurities are formed upon storage, evaluation of this parameter after storage is not necessary. Period of validity: 3 years. |
|  |  |  | The formulation is physically and chemically stable after storage for three years at 25°C in the following packaging material: - High density polyethylene pack (HDPE) For full details, please see Appendix 5. | N | Kundel P., 2017 A17960B_10279; VV-467850 |  |
| Shelf life in months (if less than 2 years) (KCP 2.7.6) | - | - | Not relevant | - | - | - |
| Wettability (KCP 2.8.1) | - | - | Not relevant | - | - | - |
| Persistence of foaming (KCP 2.8.2) | CIPAC MT 47.2 | 619 g/L cyantraniliprole FS (A17960B) | Concentration 6% in CIPAC water D: after 10 sec 0 ml after 1 min 0 ml after 3 min 0 ml after 12 min 0 ml Concentration 1.5% in CIPAC water D: after 10 sec 0 ml after 1 min 0 ml after 3 min 0 ml after 12 min 0 ml | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Suspensibility (KCP 2.8.3.1) | CIPAC MT 184 | 619 g/L cyantraniliprole FS (A17960B) | Gravimetrically Concentration 6% in CIPAC water D at 30°C: 98% Concentration 1.5% in CIPAC water D at 30°C: 97% Chemical Assay: Concentration 6% in CIPAC water D at 30°C: Cyantraniliprole : 98% Concentration 1.5% in CIPAC water D at 30°C: Cyantraniliprole : 97% | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Spontaneity of dispersion (KCP 2.8.3.2) | - | - | Not relevant | - | - | - |
| Dispersion stability (KCP 2.8.3.3) | - | - | Not relevant | - | - | - |
| Degree of dissolution and dilution stability (KCP 2.8.4) | - | - | Not relevant | - | - | - |
| Particle size distribution / nominal size range of granules (KCP 2.8.5.1.1) | - | - | Not relevant | - | - | - |
| Wet sieve test (KCP 2.8.5.1.2) | CIPAC MT 185 | 619 g/L cyantraniliprole FS (A17960B) | Less than 0.01% retained on a 75 µm sieve | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Dust content (KCP 2.8.5.2.1) | - | - | Not relevant | - | - | - |
| Particle size of dust (KCP 2.8.5.2.2) | CIPAC MT 187 | 619 g/L cyantraniliprole FS (A17960B) | d10: 0.64 μm d50: 1.63 μm d90: 5.26 μm | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Attrition (KCP 2.8.5.3) | - | - | Not relevant | - | - | - |
| Hardness and integrity (KCP 2.8.5.4) | - | - | Not relevant | - | - | - |
| Emulsifiability (KCP 2.8.6.1) | - | - | Not relevant | - | - | - |
| Emulsion stability (KCP 2.8.6.2) | - | - | Not relevant | - | - | - |
| Re-emulsifiability (KCP 2.8.6.3) | - | - | Not relevant | - | - | - |
| Flowability (KCP 2.8.7.1) | - | - | Not relevant | - | - | - |
| Pourability (KCP 2.8.7.2) | CIPAC MT 148 | 619 g/L cyantraniliprole FS (A17960B) | Pour residue: 2.8% Rinsed residue: 0.11% | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Dustability following accelerated storage (KCP 2.8.7.3) | - | - | Not relevant | - | - | - |
| Physical compatibility of tank mixes (KCP 2.9.1) | - | - | This dossier does not include recommendations for the specific mandatory tank mixing of the preparation with any other product. | - | - | - |
| Chemical compatibility of tank mixes (KCP 2.9.2) | - | - | This dossier does not include recommendations for the specific mandatory tank mixing of the preparation with any other product. | - | - | - |
| Adhesion to seeds (KCP 2.10.1) | CIPAC MT 194 | 619 g/L cyantraniliprole FS (A17960B) | Adhesion to maize seeds Retention capacity - Cyantraniliprole 96% Adhesion to sunflower seeds Retention capacity - Cyantraniliprole 101% Adhesion to oil seed rape seeds Retention capacity - Cyantraniliprole 98% Adhesion to soybean seeds Retention capacity - Cyantraniliprole 97% | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Distribution to seed (KCP 2.10.2) | CIPAC MT 175 | 619 g/L cyantraniliprole FS (A17960B) | Maize.- - Thousand grain weight : 294 g - Minimum value : 713 μg/seed - Maximum value : 1296 μg/seed - Average (loading) conc. value : 872 μg/seed - Standard deviation : 95.5 - Relative standard deviation : 10.9 % Sunflower :- - Thousand grain weight : 59 g - Minimum value : 199 μg/seed - Maximum value : 497 μg/seed - Average (loading) conc. value : 298 μg/seed - Standard deviation : 55.5 - Relative standard deviation : 18.6 % Oil seed rape :- - Thousand grain weight : 4.3 g - Minimum value : 32.0 μg/seed - Maximum value : 110 μg/seed - Average (loading) conc. value : 58.2 μg/seed - Standard deviation : 11.9 - Relative standard deviation : 20.4 % Soybean :- - Thousand grain weight : 117 g - Minimum value : 120 μg/seed - Maximum value : 277 μg/seed - Average (loading) conc. value : 174 μg/seed - Standard deviation : 28.8 - Relative standard deviation : 16.5 % | Y | Khot S., 2014 A17960B_10046; VV-406754 | Accepted. |
| Other/special studies (KCP 2.11/01) | - | A17960A | Immediately after use, clean the application equipment thoroughly. Drain the system completely and rinse the apparatus with excess water until it is visually clean. | N | Naegele M., 2011; 10460657 | The applicant has provided a statement regarding the cleaning procedure. According to Regulation (EU) No. 284/2013, paragraph 4.2 the effectiveness of the recommended tank cleaning procedures must be addressed. Data submitted must demonstrate that residues of the plant protection product do not remain in the spray tank after cleaning such that there is a risk to the operator or crops. The method used must be fully reported and justified. Statement is not sufficient and a study report should be available. Such conclusion was indicated during the evaluation of the zonal evaluation of Fortenza performed for purposes of authorization as a seed treatment in oilseed rape, finalized in July 2020 however for Fortenza, double rinsing with water seems to be sufficient as equipment washing procedure based on the results of the pourability test performed with A17960B (method CIPAC MT 148, please refer to annex point KCP 2.8.7.2), which showed accepted rinsed residue (0.11 %) after rinsing with water. Taking in to account the results of pourability study and the statement of the applicant, double rinsing with tap water is recommended. |
| Other/special studies (KCP 2.11/02) | - | A17960A | The effectiveness of the cleaning procedure can be inspected visually. If no residue is left in the equipment, all formulation has been removed. | N | Naegele M., 2011; 10460655 | Please refer to the KCP 2.11/01. |

# Section 3 is presented as a separate document

Please refer to the separate file “dRR Part B3”.

# Section 4: Further information on the plant protection product

## Packaging and Compatibility with the Preparation (KCP 4.4)

Table 4.1-1:	Packaging information for 5 L canister

| Type | Description |
| --- | --- |
| Material: | high density polyethylene (HDPE) |
| Shape/size: | canister / 190mm x 135mm x 315mm (Length x Width x Height) |
| Opening , closure and seal: | Screw cap closure (63 mm diameter) with induction heat seal or compression wad and tamper evident ring |
| Manner of construction | Blow-molded |
| UN/ADR | compliant |

Table 4.1-2:	Packaging information for 10 L canister

| Type | Description |
| --- | --- |
| Material: | high density polyethylene (HDPE) |
| Shape/size: | canister / 240mm x 180 mm x 375 mm (Length x Width x Height) |
| Opening , closure and seal: | Screw cap closure (63 mm diameter) with induction heat seal or compression wad and tamper evident ring |
| Manner of construction | Blow-molded |
| UN/ADR | compliant |

Table 4.1-3:	Packaging information for 20 L drum

| Type | Description |
| --- | --- |
| Material: | high density polyethylene (HDPE) |
| Shape/size: | drum / 295mm x 245 mm x 380 mm (Length x Width x Height) |
| Opening , closure and seal: | 2 x Screw cap closure DIN 60 with induction heat seal or compression wad and tamper evident ring |
| Manner of construction | Blow-molded |
| UN/ADR | compliant |

Table 4.1-4:	Packaging information for 50 L drum

| Type | Description |
| --- | --- |
| Material: | high density polyethylene (HDPE) |
| Shape/size: | drum / 380 mm x 605 mm (Diameter x Height) |
| Opening , closure and seal: | 2 x Screw cap closure DIN 60 with induction heat seal or compression wad and tamper evident ring |
| Manner of construction | Blow-molded |
| UN/ADR | compliant |

Table 4.1-5:	Packaging information for 200 L drum

| Type | Description |
| --- | --- |
| Material: | High density polyethylene pack with additional barrier material (polyamide) (HDPE/PA) |
| Shape/size: | drum / 580mm x 960mm (diameter x height) |
| Opening , closure and seal: | 2 x Screw cap closure DIN 60 with induction heat seal or compression wad and tamper evident ring |
| Manner of construction | Blow-molded |
| UN/ADR | compliant |

Table 4.1-6:	Packaging information for 1000 L IBC

| Type | Description |
| --- | --- |
| Material: | High density polyethylene pack with additional barrier material (polyamide) (HDPE/PA) |
| Shape/size: | container / 1200mm x 1000mm x 1160mm (length x width x height) |
| Opening , closure and seal: | Top: 150mm with gasket and facility for tamper evidenceBottom: 50mm valve with screw cap |
| Manner of construction | Blow-molded |
| UN/ADR | compliant |

The packaging for the product A17960B is in compliance with all relevant UN and ADR requirements.

Stability of the packaging material has been tested during the storage stability study done according to GIFAP monograph 17.

No significant adverse effects of the product to the stability of the packaging material have been noticed.

It is concluded the packaging material will be fully resistant to the product A17960B for up to 2 years under normal storage conditions. For details please see Appendix 3 – 4.

- Lists of data considered in support of the evaluation

List of data submitted by the applicant and relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| KCP Section 2 / 01 & KCP 2.6 / 01 | Abraham N. | 2014 | A17960B - Chemical Characterization before Storage of Batch SMU3HL016 Syngenta Syngenta Biosciences Pvt. Ltd., Ilhas Goa, India, SMG12351 GLP not published Syngenta File No A17960B_10050 | N | SYN |
| KCP Section 2 / 02 | Das R. | 2011 | A17960B - Chemical characterization of batch SMU1AP002 Syngenta Syngenta Crop Protection, Münchwilen, Switzerland, 122194 GLP not published Syngenta File No A17960B_10014 | N | SYN |
| KCP 2.1 / 01 & KCP 2.5 / 01 & KCP 2.7 / 02 & KCP 2.8.2 / 01 & KCP 2.8.3 / 01 & KCP 2.8.5.1 / 01 & KCP 2.8.5.2 / 01 & KCP 2.8.7 / 01 & KCP 2.10 / 01 | Khot S. | 2014 | A17960B - Physical and Technical Properties of Batch SMU3HL016.pdf Syngenta Syngenta Biosciences Pvt. Ltd., Ilhas Goa, India, SMG12352 GLP not published Syngenta File No A17960B_10046 | N | SYN |
| KCP 2.2 / 01 & KCP 2.3 / 02 | Jackson W. | 2011 | A17960A - Safety study Syngenta Syngenta Technology & Projects, Huddersfield, United Kingdom, HT11/504 GLP not published Syngenta File No A17960A_10015 | N | SYN |
| KCP 2.3 / 01 | Jackson W. | 2011a | A17960B - Flash point Syngenta Syngenta Technology & Projects, Huddersfield, United Kingdom, HT11/516 GLP not published Syngenta File No A17960B_10008 | N | SYN |
| KCP 2.7 / 01 | Kundel P. | 2014 | A17960B - Storage Stability and Shelf Life Statement (2 Weeks 54 -C) in Packaging Made of HDPE according to CIPAC MT 46.3 Syngenta Syngenta Biosciences Pvt. Ltd., Ilhas Goa, India, 300016939 Not GLP not published Syngenta File No A17960B_10051 | N | SYN |
| KCP 2.7 / 03 | Kundel P. | 2016 | A17960B - Storage Stability and Shelf Life Statement (2 Years 25 -C) in Packaging Made of HDPE Syngenta Syngenta Crop Protection, Münchwilen, Switzerland, 300056658 Not GLP not published Syngenta File No A17960B_10264 | N | SYN |
| KCP 2.7 / 04 | Kundel P. | 2017 | A17960B - Storage Stability and Shelf Life Statement (3 Years 25 °C) in Packaging Made of HDPE Syngenta Syngenta Crop Protection, Münchwilen, Switzerland, 300083560 Not GLP not published Syngenta File No A17960B_10279; VV-467850 | N | SYN |
| KCP 2.11 / 01 | Naegele M. | 2011 | A17960A; Procedure for Cleaning Application Equipment; Syngenta Crop Protection, Münchwilen, Switzerland, 300056658 Not GLP not published Syngenta File No 10460657 | N | SYN |
| KCP 2.11 / 02 | Naegele M. | 2011 | A17960A; Effectiveness of the Cleaning Procedure for the Application Equipment; Syngenta Crop Protection, Münchwilen, Switzerland, 300056658 Not GLP not published Syngenta File No 10460655 | N | SYN |

List of data submitted or referred to by the applicant and relied on, but already evaluated at EU peer review

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data submitted by the applicant and not relied on

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

List of data relied on and not submitted by the applicant but necessary for evaluation

| Data point | Author(s) | Year | Title Company Report No. Source (where different from company) GLP or GEP status Published or not | Vertebrate study Y/N | Owner |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |

- Additional data on the physical, chemical and technical properties of the active substance

No additional data have been generated on the active substance Cyantraniliprole.

Appendix 3:	Storage stability data before and after storage for two weeks at 54°C in HDPE packaging

Table A3-1: Content of active ingredients before and after storage for two weeks at 54°C in HDPE packaging

| Active Ingredient | Storage Conditions | Content of control sample | Content of test sample |
| --- | --- | --- | --- |
| cyantraniliprole | Initial | 619 g/L | - - |
| cyantraniliprole | 2 weeks below -10 °C | 620 g/L | - - |
| cyantraniliprole | 2 weeks 54 °C | - - | 621 g/L |

Observations and Conclusion

All the values are well within the analytical error and show no decomposition.

Table A3-2: Physical and technical properties before and after storage for two weeks at 54°C in HDPE packaging

| Test Description | Method | Initial Results | Results after 2 weeks 54 °C |
| --- | --- | --- | --- |
| Color | Visual | cream colored | cream colored |
| Odor | Organoleptic | chalky | chalky |
| Physical State | Visual | liquid | liquid |
| Appearance | Visual | - - - | no claying, easily redispersible |
| pH Value (at 25oC) Concentration: 1 % Deionized Water | CIPAC MT 75.3 | 5.3 | 5.2 |
| pH Value (at 25oC) Undiluted | CIPAC MT 75.3 | 4.3 | 4.0 |
| Relative Density Temperature: 20 °C | OECD 109 | 1.255 g/cm3 | 1.257 g/cm3 |
| Wet Sieve Test Sieve Size: 75 µm | CIPAC MT 185 | < 0.01 % | 0.02 % |
| Pourability Pour Residue Rinsed Residue | CIPAC MT 148 | 2.8 % 0.11 % | 2.3 % 0.12 % |
| Suspensibility Concentration: 6 % CIPAC Water D Temperature: 30 °C Waiting Period: 30 min. cyantraniliprole | CIPAC MT 184 (chemical assay) | 98 % | 97 % |
| Suspensibility Concentration: 1.5 % CIPAC Water D Temperature: 30 °C Waiting Period: 30 min. cyantraniliprole | CIPAC MT 184 (chemical assay) | 97 % | 98 % |
| Adhesion to Seeds (maize) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 96 % | 97 % |
| Adhesion to Seeds (sunflower) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 101 % | 96 % |
| Adhesion to Seeds (oil seed rape) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 98 % | 99 % |
| Adhesion to Seeds (soybean) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 97 % | 99 % |

Observations and Conclusion

No significant changes of the physical and technical properties were found after storage.

Table A3-3: Packaging Evaluation after storage for two weeks at 54°C in HDPE packaging

| Evaluation Criteria | Results after 2 weeks 54 °C |
| --- | --- |
| Color change of the packaging | none |
| Odor (noticeable before opening the packaging) | none |
| Panelling of the test container | none |
| Ballooning of the test container | none |
| Pimples on the test container | none |
| Cracks in the test container | none |
| Tightness of the test container | tight |
| Reclosability of closure | reclosable |
| Tightness of closure | tight |
| Weight change (gross weight) | 0.03 % weight loss |
| Permeation through the container walls | none |

Observations and Conclusion

The packaging material proved to be resistant to its content.

Appendix 4:	Storage stability data before and after storage for two years at 25°C in HDPE packaging

Table A3-1: Content of active ingredients before and after storage for two years at 25°C in HDPE packaging

| Active Ingredient | Storage Conditions | Content of control sample | Content of test sample |
| --- | --- | --- | --- |
| cyantraniliprole | Initial | 619 g/L | - - |
| cyantraniliprole | 2 weeks below -10 °C | 620 g/L | - - |
| cyantraniliprole | 2 weeks 54 °C | - - | 623 g/L |

Observations and Conclusion

All the values are well within the analytical error and show no decomposition.

Table A3-2: Physical and technical properties before and after storage two years at 25°C in HDPE packaging

| Test Description | Method | Initial Results | Results after 2 years 25 °C |
| --- | --- | --- | --- |
| Color | Visual | cream colored | cream colored |
| Odor | Organoleptic | chalky | chalky |
| Physical State | Visual | liquid | liquid |
| Appearance | Visual | - - - | no claying, easily redispersible |
| pH Value (at 25oC) Concentration: 1 % Deionized Water | CIPAC MT 75.3 | 5.3 | 4.4 |
| pH Value (at 25oC) Undiluted | CIPAC MT 75.3 | 4.3 | 3.4 |
| Relative Density Temperature: 20 °C | OECD 109 | 1.255 g/cm3 | 1.257 g/cm3 |
| Wet Sieve Test Sieve Size: 75 µm | CIPAC MT 185 | < 0.01 % | 0.02 % |
| Pourability Pour Residue Rinsed Residue | CIPAC MT 148 | 2.8 % 0.11 % | 2.1 % 0.10 % |
| Suspensibility Concentration: 6 % CIPAC Water D Temperature: 30 °C Waiting Period: 30 min. cyantraniliprole | CIPAC MT 184 (chemical assay) | 98 % | 98 % |
| Suspensibility Concentration: 1.5 % CIPAC Water D Temperature: 30 °C Waiting Period: 30 min. cyantraniliprole | CIPAC MT 184 (chemical assay) | 97 % | 97 % |
| Adhesion to Seeds (maize) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 96 % | 99 % |
| Adhesion to Seeds (sunflower) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 101 % | 98 % |
| Adhesion to Seeds (oil seed rape) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 98 % | 98 % |
| Adhesion to Seeds (soybean) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 97 % | 96 % |

Observations and Conclusion

No significant changes of the physical and technical properties were found after storage.

Table A3-3: Packaging Evaluation after storage for two years at 25°C in HDPE packaging

| Evaluation Criteria | Results after 2 years 25 °C |
| --- | --- |
| Color change of the packaging | none |
| Odor (noticeable before opening the packaging) | none |
| Panelling of the test container | none |
| Ballooning of the test container | none |
| Pimples on the test container | none |
| Cracks in the test container | none |
| Tightness of the test container | tight |
| Reclosability of closure | reclosable |
| Tightness of closure | tight |
| Weight change (gross weight) | 0.08 % weight loss |
| Permeation through the container walls | none |

Observations and Conclusion

The packaging material proved to be resistant to its content.

Appendix 5:	Storage stability data before and after storage for three years at 25°C in HDPE packaging

Table A5-1: Content of active ingredients before and after storage for three years at 25°C in HDPE packaging

| Active Ingredient | Storage Conditions | Content of control sample | Content of test sample |
| --- | --- | --- | --- |
| Cyantraniliprole | Initial | 619 607 g/l | - - |
| Cyantraniliprole | 3 years below -10 °C | 604 600 g/l | - - |
| Cyantraniliprole | 3 years 25 °C | - - | 606 598 g/l |

Observations and Conclusion

All the values are well within the analytical error and show no decomposition.

Table A5-2: Physical and technical properties before and after storage three years at 25°C in HDPE packaging

| Test Description | Method | Initial Results | Results after 3 years 25 °C |
| --- | --- | --- | --- |
| Color | Visual | cream colored | cream colored |
| Odor | Organoleptic | chalky | chalky |
| Physical State | Visual | liquid | liquid |
| Appearance | Visual | - - - | no claying, easily redispersible |
| pH Value at 25oC Concentration: 1 % Deionized Water | CIPAC MT 75.3 | 5.3 5.5 | 4.4 4.5 |
| pH Value at 25oC Undiluted | CIPAC MT 75.3 | 4.3 4.7 | 3.2 3.5 |
| Relative Density Temperature: 20 °C | OECD 109 | 1.2554 g/cm3 | 1.2576 g/cm3 |
| Wet Sieve Test Sieve Size: 75 µm | CIPAC MT 185 | < 0.01 % | 0.01 % |
| Pourability Pour Residue Rinsed Residue | CIPAC MT 148 | 2.86 % 0.11 0.10 % | 2.34 % 0.11 0.08 % |
| Suspensibility Concentration: 6 % CIPAC Water D Temperature: 30 °C Waiting Period: 30 min. cyantraniliprole | CIPAC MT 184 (chemical assay) | 98 97 % | 97 96 % |
| Suspensibility Concentration: 1.5 % CIPAC Water D Temperature: 30 °C Waiting Period: 30 min. cyantraniliprole | CIPAC MT 184 (chemical assay) | 97 % | 97 96 % |
| Adhesion to Seeds (maize) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 96 94% | 97 98% |
| Adhesion to Seeds (sunflower) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 101 99 % | 99 98 % |
| Adhesion to Seeds (oil seed rape) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 98 99 % | 98 99 % |
| Adhesion to Seeds (soybean) Retention Capacity of: cyantraniliprole | CIPAC MT 194 (chemical assay) | 97 100 % | 99 98 % |

Observations and Conclusion

No significant changes of the physical and technical properties were found after storage.

Table A5-3: Packaging Evaluation after storage for three years at 25°C in HDPE packaging

| Evaluation Criteria | Results after 3 years 25 °C |
| --- | --- |
| Color change of the packaging | none |
| Odor (noticeable before opening the packaging) | none |
| Panelling of the test container | none |
| Ballooning of the test container | none |
| Pimples on the test container | none |
| Cracks in the test container | none |
| Tightness of the test container | tight |
| Reclosability of closure | reclosable |
| Tightness of closure | tight |
| Weight change (gross weight) | 0.08 % weight loss |
| Permeation through the container walls | none |

Observations and Conclusion

The packaging material proved to be resistant to its content.