REGISTRATION REPORT Part B Section 10 Assessment of the relevance of metabolites in groundwater Detailed summary of the risk assessment

Product code: SHA 148000 A Product name(s): METROPOLITAN Chemical active substance(s): Metazachlor, 500 g/L

Central Zone Zonal Rapporteur Member State: Poland

CORE ASSESSMENT (authorization)

Applicant: XXXX Submission date: October 2022 Evaluation date: July 2023 MS Finalisation date: October 2023

Version history

| When | What |
| --- | --- |
| July 2023 | Version evaluated by zRMS PL |
| September 2023 | Version revised to take into account comments of cMSs and the applicant |

Table of Contents

10	Relevance of metabolites in groundwater	4

10.1	General information	4

10.2	Relevance assessment of metabolites of Metazachlor	5

10.2.1	STEP 1: Exclusion of degradation products of no concern	8

10.2.2	STEP 2: Quantification of potential groundwater contamination	8

10.2.3	STEP 3: Hazard assessment – identification of relevant metabolites	10

10.2.3.1	STEP 3, Stage 1: screening for biological activity	10

10.2.3.2	STEP 3, Stage 2: screening for genotoxicity	10

10.2.3.3	STEP 3, Stage 3: screening for toxicity	10

10.2.4	STEP 4: Exposure assessment – threshold of concern approach	11

10.2.5	STEP 5: Refined risk assessment	12

Appendix 1	Lists of data considered in support of the evaluation	15

Appendix 2	Additional information	15

# Relevance of metabolites in groundwater

## General information

The metabolites 479M04, 479M08, 479M09, 479M11, 479M12 are predicted to occur in groundwater at concentrations above 0.1 µg/L (see section dRR Part B8). Assessment of the relevance of these metabolites according to the stepwise procedure of the EC guidance document SANCO/221/2000 –rev.11 is therefore required.

General information on the metabolites is provided in Table 10.1-1. The impact of the relevance assessment on whether a particular GAP use leads to acceptable risk or not is presented in the summary of the cGAP evaluation in chapter 8.7 of the dRR Part B, Section 8 (Environmental fate and behaviour).

Table 10.1-1:	General information on the metabolite(s)

| Name of active substance | Metabolite name and code | Structural/molecular formula | Trigger for relevance assessment |  |
| --- | --- | --- | --- | --- |
| Metazachlor | N-(2,6-dimethylphenyl)-N-(1H-pyrazol-1- ylmethyl)oxalamide Code: 479M04/BH 479-4 |  | Max PECgw Based on: | 4.700 μg/L FOCUS PEARL v5.5.5, Hamburg WOSR pre emergence at 750 g/ha trienial application |
|  | N-(2,6-dimethylphenyl)-N-(1H-pyrazol-1-ylmethyl)aminocarbonylmethylsulfonic acid sodium N-(2,6-dimethylphenyl)-N-(1H-pyrazol-1-ylmethyl)aminocarbonylmethylsulfonat Code: 479M08/BH 479-8/BH 479- 18 |  | Max PECgw Based on: | 9.875 μg/L 7.701 μg/L FOCUS PEARL v5.5.5 Châteaudun FOCUS PELMO v6.6.4, Hamburg WOSR pre emergence at 750 g/ha trienial application |
|  | N-(2,6-dimethylphenyl)-N-(1H-pyrazol-1- ylmethyl)aminocarbonylmethylsulfinyl acetic acid Code: 479M09/BH 479-09 |  | Max PECgw Based on: | 0.967 μg/L FOCUS PELMO v6.6.4, Hamburg WOSR pre emergence at 750 g/ha trienial application |
|  | methyl N-(2,6-dimethylphenyl)-N-(1H-pyrazol-1-ylmethyl)aminocarbonylmethylsulfoxide Code: 479M11/BH 479-11 |  | Max PECgw Based on: | 0.959 μg/L FOCUS PELMO v6.6.4, Piacenza WOSR pre emergence at 750 g/ha trienial application |
|  | N-[(2-hydroxycarbonyl-6-methyl)phenyl]-N- (1H-pyrazol-1-ylmethyl)oxalamide Code: 479M12/BH 479-12 |  | Max PECgw Based on: | 11.334 μg/L FOCUS PELMO v6.6.4, Châteaudun WOSR pre emergence at 750 g/ha trienial application |

## Relevance assessment of metabolites of Metazachlor

Summary:

The relevance of the groundwater metabolites 479M04, 479M08, 479M09, 479M11 and 479M12 has already been assessed and the assessment agreed at EU level (see Metazachlor Addendum – Confirmatory Data, 2013 and EFSA conclusion - EFSA Journal 2017;15(6):4833), but the relevance assessment is not applicable for the GAP and groundwater scenarios considered in this dRR (i.e., the conclusions reached at Step 4 and 5 of the relevance assessment made at the EU-level are not valid with regard to the PECgw calculated for the GAP and groundwater scenarios considered in this dRR). Therefore, the assessment and conclusions are presented here (see 10.1.4-10.1.5). Metabolites 479M04, 470M08 and 479M12 are not considered toxicologically relevant and metabolites 479M09 and 479M11 are toxicologically relevant according to the criteria laid down in the EC guidance document SANCO/221/2000 –rev.11. A summary of the relevance assessment is given in Table 10.2-2 and the corresponding studies are listed in the corresponding sections.

Table 10.2-2:	Summary of the relevance assessment for metabolites of Metazachlor

|  | Assessment step |  | Result of assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 479M04 | 479M08 | 479M09 | 479M11 | 479M12 |
|  | STEP 1 |  | Metabolite of no concern? | yes |  |  |  |  |
| Quantification of groundwater contamination | STEP 2 |  | Max PECgw (µg/L) | 4.700 | 9.875 μg/L 7.701 μg/L | 0.967 | 0.959 | 11.334 |
|  |  |  | Based on | FOCUS PEARL v5.5.5, Hamburg WOSR pre emergence at 750 g/ha trienial application | FOCUS PEARL v5.5.5 Châteaudun FOCUS PELMO v6.6.4, Hamburg WOSR pre emergence at 750 g/ha trienial application | FOCUS PELMO v6.6.4, Hamburg WOSR pre emergence at 750 g/ha trienial application | FOCUS PELMO v6.6.4, Piacenza WOSR pre emergence at 750 g/ha trienial application | FOCUS PELMO v6.6.4, Châteaudun WOSR pre emergence at 750 g/ha trienial application |
| Hazard assessment | STEP 3 | Stage 1 | Biological activity comparable to the parent? | No |  |  |  |  |
|  |  | Stage 2 | Genotoxic properties of metabolite |  |  |  |  |  |
|  |  | Stage 3 | Toxic properties of metabolite; | No |  | Yes |  |  |
|  |  |  | Classification of parent | Carc. Cat. 2 (H351 H315) |  |  |  |  |
|  |  |  | Classification of metabolite | - | - | Carc. Cat. 2 (H315) |  | - |
| Consumer health risk assessment | STEP 4 |  | Estimated consumer exposure via drinking water and other sources; threshold of concern approach | not acceptable (>0.75 µg/L) | not acceptable (>0.75 µg/L) | - | - | not acceptable (>0.75 µg/L) |
|  | STEP 5 |  | Refined risk assessment | Acceptable | Acceptable | - | - | Acceptable |
|  |  |  | Predicted exposure (NOAEL of metabolites) (mg/kg bw/d) | 469 | 282 | - | - | 381 |
|  |  |  | MAC of Parent (conservative approach) (µg/L) | 240 |  | - | - | 240 |

*	N/A: not applicable

### STEP 1: Exclusion of degradation products of no concern

All the metabolites of interest contain a phenyl and azole ring and therefore are not automatically of no concern. In addition, there is insufficient information available on their possible natural occurrence.

### STEP 2: Quantification of potential groundwater contamination

PECgw calculations after leaching from soil for 479M04, 479M08, 479M09, 479M11 and 479M12 were performed (see Part B, Section 8, chapter 8.7). The uses for which concentrations of metabolites 479M04, 479M08, 479M09, 479M11 and 479M12 were considered to exceed 0.1 µg/L are listed in Table 10.2-2. Details are given in Part B, Section 8, chapter 8.7.

According to Metazachlor Addendum – Confirmatory Data (Mammalian Toxicology and Relevance of Groundwater Metabolites), 2013 the PECgw values for 479M09, 479M11, and 479M12 from FOCUS models, probably due to the use of laboratory DT50 input values, because no DT50 from field was derived. This conclusion was based on extensive groundwater monitoring data which indicated that the FOCUSgw models predicted concentrations were significantly higher than observed in monitored samples. The higher tier assessment ground water monitoring studies are done, one in Germany and one in France (9.6.3/1, Schneider M., Penning H., 2011a). The agreed endpoints for 479M09, 479M11, and 479M12 below are based on observed data from the above 2 higher tier monitoring studies.

| Metabolite | PECgw (µg/L) from monitoring studies | PECgw (µg/L) from FOCUS simulations |
| --- | --- | --- |
| 479M04 | <10 | 4.700 |
| 479M08 | <10 | 9.875 μg/L 7.701 μg/L |
| 479M12 | <1.5 | 11.334 |
| 479M09 | <0.1 | 0.967 |
| 479M11 | <0.1 | 0.959 |

According to Metazachlor Addendum – Confirmatory Data (Mammalian Toxicology and Relevance of Groundwater Metabolites), 2013, these FOCUS PECgw were adjusted with the transfer factors derived from the monitoring studies according to Equation below:

PECgw (adj.) = PECgw(Focus)/TF

With,

PECgw (adj.) 	PECgw adjusted with transfer factors [µg/L]

PECgw (Focus)	PECgw calculated for the FOCUS scenarios [µg/L]

TF 			 transfer factor derived from monitoring studies [ -]

The transfer factors for metabolites BH 479-9, BH 479-11 and BH 479-12 are 47, 36 and 6, respectively, whereas no factors are used for metazachlor, BH 479-4 and BH 479-8.

| Metabolite | PECgw (µg/L) from monitoring studies | PECgw (µg/L) from FOCUS simulations | PECgw (µg/L) after adjust |
| --- | --- | --- | --- |
| 479M04 | <10 | 4.700 | 4.700 |
| 479M08 | <10 | 9.875 μg/L 7.701 μg/L | 9.875 μg/L 7.701 μg/L |
| 479M12 | <1.5 | 11.334 | 1.889 |
| 479M09 | <0.1 | 0.967 | 0.02 |
| 479M11 | <0.1 | 0.959 | 0.03 |

The agreed endpoints for M09, M11, and M12 below are based on observed data from higher tier monitoring studies. Hence, FOCUSgw models produced unrealistic PECgw values for M09, M11, and M12, probably due to the use of laboratory DT50 input values.

The PECgw values for metabolites M04 and M08 range between 1 – 10 µg/L based on the monitoring studies conducted on Germany and France. The max value for 479M12 was above 10 µg/L but only in Châteaudun scenario in WOSR in pre-emergence at 750 g as/ha based on peak groundwater monitoring concentrations, and for metabolites 479M09 and 479M11 the PECgw values are below 0.1 µg/L (also based on results of groundwater monitoring).

Metabolites 479M04, 479M08 and 479M12 should proceed to step 3.

The revised PECgw indicate metabolites 479M09 and 479M11 are not predicted to occur in groundwater above the parametric drinking water limit of 0.1 µg/L and are not relevant for groundwater risk assessment (pass at step 2). Observations from higher tier monitoring indicated that 479M11 does not leach at concentrations greater than 0.1 μg/L, even at vulnerable sites where significant concentrations of the three main metabolites were regularly observed, and that 479M09 is also very unlikely to leach at concentrations greater than 0.1 μg/L. The exception to this being two individual samples (highest 0.28 µg/L) at particularly vulnerable French sites (Treigny and Sommercaise) after unusually high rainfall. For M09, even at these vulnerable sites with very heavy rainfall at certain times of the year (> 98th percentile weather events), annual average concentrations were always observed to be less than 0.1 ug/L. However, given these (albeit exceptional) observations, further comment on exposure is included under the overall conclusion on relevance below.

Review Comments:

The evaluation for metazachlor was conducted in accordance of EFSA Scientific Report (2008) 145, 1-132. Moreover, the data from evaluation of confirmatory data regarding the groundwater exposure (Technical Annex to Addendum – revised August 2016 and EFSA Peer review of the pesticide risk assessment for the active substance metazachlor in light of confirmatory data submitted– April 2017) provide new data concerning the concentrations of the metabolites 479M09, 479M11 and 479M12 in groundwater, which were taken to consideration in the risk assessment.

After comments by EFSA and MS, the RMS - UK revisited degradation rates for metabolites of metazachlor.

In Technical Annex to Addendum (Revised August 2016) is present updated evaluation of assessment of concentration of metzachlor metabolites in groundwater under consideration of monitoring results.

Monitoring data

Overall, the relevance and vulnerability of the monitoring sites in Germany in 4 sites were accepted. The quality of the data fulfils the criteria set out in the FOCUS guidance (2009) on groundwater monitoring.

Concentrations of metabolites in groundwater in the targeted monitoring (according to EFSA Journal 2017;15(6):4833) are summarised in table below.

| Metabolite (4 sites, 48 samples) | Maximum PECGW value [µg/L] |
| --- | --- |
| BH479-4 (479M04) | 1.8 |
| BH479-8 (479M08) | 6.76 |
| BH479-9 (479M09) | < 0.05 |
| BH479-11 (479M11) | < 0.05 |
| BH479-12 (479M12) | 0.16 |

The monitoring data are not agreed between Member States and decision on their appropriateness as a higher tier risk assessment is still pending in the EU. Therefore, Member States are encouraged to make their own decision on relevance of German monitoring studies for their conditions and the use of groundwater monitoring as higher tier (hydrogeological data of targeted sites in France were classify as insufficient).

The PECGW calculations for metazachlor and its metabolites were provided by the Applicant and are considered acceptable.

Based on the results of FOCUS groundwater calculations (application of product every third year) for metazachlor and higher tier refinement based on monitoring data for 479M09 and 479M11, which are of toxicological concern, do not exceed the regulatory trigger of 0.1 µg/L at 1 m depth in any of the scenarios.

Based on the results of FOCUS groundwater calculations (application of product every third year) the metabolites 479M04, 479M08 and 479M12 (except use in winter OSR pre-emergence), were found at concentrations above 0.1 µg/L but below 10 µg/L. The monitoring data submitted as part of the confirmatory data show that none of these metabolites is expected to exceed 10 µg/L in the most vulnerable soils where oilseed rape is grown.

### STEP 3: Hazard assessment – identification of relevant metabolites

#### STEP 3, Stage 1: screening for biological activity

One of the key stages in the assessment of potential relevance of a metabolite is the determination of biological activity. In the context of metazachlor, which is a broad-spectrum herbicide, it is whether the metabolites listed in the table above retain herbicidal activity in comparison to the parent.

In the guidance document Sanco/221/2000 –rev.10 it states that in screening assays it will often not be possible to determine and compare the biological activity of a parent molecule and its metabolites with great precision, and this will also not be necessary in most cases. As a line of orientation, it should be sufficient to demonstrate that the biological activity of a metabolite is clearly less than 50% of the activity of the parent molecule. Otherwise, the biological activity should be considered as “comparable”.

In the original DAR, Vol 3 section B.3.1.5 data were reported by BASF on all 5 metabolites. BASF supplied data on all these metabolites and non-relevance (for biological activity) was demonstrated for all five metabolites. Non-relevance for biological activity was demonstrated for both of these metabolites. As stage 1 of Step 3 is passed, the three gw metabolites of concern, 479M04, 479M08 and 479M12 must be further screened in stage 2 of Step 3.

#### STEP 3, Stage 2: screening for genotoxicity

None of these three gw metabolites of metazachlor (479M04, 479M08 and 479M12) are considered to be genotoxic based on a standard battery of genotoxicity tests (as reported in the DAR section B.6.8.1 and DAR addendum 4, Sept 2007). As stage 1 of Step 3 is passed, the three gw metabolites of concern, 479M04, 479M08 and 479M12 must be further screened in stage 3 of Step 3.

#### STEP 3, Stage 3: screening for toxicity

According to section B.6.8.2.1 of Metazachlor Addendum – Confirmatory Data (Mammalian Toxicology and Relevance of Groundwater Metabolites), 2013, all five metabolites were already evaluated in the DAR on metazachlor and found to be not of toxicological relevance. However, in light of the recent decision to classify metazachlor with Carc. Cat. 2 (H351 H315) under the CLP Regs (RAC 2011), the toxicological profile (especially with regard to effects on liver and kidney) of these metabolites and their similarity to the parent compound were investigated further to determine whether any of the early toxicological events/effects (or their absence) seen in rat liver and mouse kidney with metazachlor are shared by its groundwater metabolites.

Stage 3 of Step 3 is aimed at the question of whether a metabolite has certain toxicological properties, which - from a regulatory perspective - qualify for considering it “relevant”. The toxicological profile and relevance of each metabolite have been considered in detail in Section B.6.8.2.1 of Metazachlor Addendum – Confirmatory Data (Mammalian Toxicology and Relevance of Groundwater Metabolites), 2013, above of this confirmatory data addendum, and are presented again in a summary form below.

Consideration of Toxic (T) and Very Toxic (T+)

On the basis of acute toxicity and repeated dose toxicity studies conducted with metazachlor and its three gw metabolites of concern, classification as toxic or very toxic for acute or repeated dose toxicity is not proposed for any of the three gw metabolites of concern (479M04, 479M08 and 479M12) or the parent; so 479M04, 479M08 and 479M12 are not considered as relevant in relation to these endpoints.

Consideration of reproductive toxicity

Metazachlor is not classified for reproductive toxicity. Rat developmental toxicity studies conducted with 479M04, and 479M08 indicate that these metabolites do not affect development. Other studies (mainly repeated dose toxicity studies) conducted with each of these three metabolites indicate that 479M4, 479M08 and 479M12 are less toxic than parent metazachlor. On this basis, it can be concluded that 479M4, 479M08 and 479M12 are not relevant in relation to reproductive toxicity.

Consideration of carcinogenicity

Metazachlor is classified as a category 2 carcinogen (under the CLP Regs) mainly on the basis of liver tumours in Wistar rats. These tumours are preceded by liver hypertrophy and P450 (mainly 2B1 and 2B2) enzyme induction.

Acute toxicity, repeated dose toxicity and mechanistic studies conducted with 479M04 show that 479M04 is less toxic than parent metazachlor and causes no hypertrophy or P450 (2B1 and 2B2) enzyme induction in the liver of Wistar rats and no damage or hyperplastic response in the kidney of CD-1 mice. On the basis of these results, it can be concluded that metabolite 479M04 appears not to share the early toxicological events/effects seen in rat liver with metazachlor.

Hence, 479M04 is unlikely to possess the carcinogenic potential of the parent compound. Overall, it can be concluded that metabolite 479M04 is ‘non-relevant’ in relation to carcinogenicity and in relation to toxicity in general and should proceed to step 4 of the groundwater assessment/decision tree.

Acute toxicity, repeated dose toxicity and mechanistic (mouse kidney) studies conducted with 479M08 show that 479M08 is less toxic than parent metazachlor and causes no increased weight or hypertrophy in the liver of Wistar rats and no damage or hyperplastic response in the kidney of CD-1 mice. On the basis of these results, it can be concluded that metabolite 479M08 appears not to share the early toxicological events/effects seen in rat liver with metazachlor. Hence, 479M08 is unlikely to possess the carcinogenic potential of the parent compound. Overall, it can be concluded that metabolite 479M08 is ‘non-relevant’ in relation to carcinogenicity and in relation to toxicity in general and should proceed to step 4 of the groundwater assessment/decision tree.

Acute toxicity, repeated dose toxicity and mechanistic studies conducted with 479M12 show that 479M12 is less toxic than parent metazachlor and causes no hypertrophy or P450 (2B1 and 2B2) enzyme induction in the liver of Wistar rats and no damage or hyperplastic response in the kidney of CD-1 mice. On the basis of these results, it can be concluded that metabolite 479M12 appears not to share the early toxicological events/effects seen in rat liver with metazachlor. Hence, 479M12 is unlikely to possess the carcinogenic potential of the parent compound. Overall, it can be concluded that metabolite 479M12 is ‘non-relevant’ in relation to carcinogenicity and in relation to toxicity in general and should proceed to step 4 of the groundwater assessment/decision tree.

### STEP 4: Exposure assessment – threshold of concern approach

As noted under Step 2 above, metabolites 479M04, 479M08 and 479M12 are identified as requiring a refined risk assessment (Step 5) as their highest gw concentrations is above the threshold of concern of 0.75 µg/L (highest concentrations predicted were 4.700, 7.701 and 1.889 (after adjust) µg/L for 479M04, 479M08 and 479M12, respectively). Therefore, step 4 is considered unacceptable, and a refined risk assessment (step 5) is required for these metabolites.

### STEP 5: Refined risk assessment

As noted under Step 4, metabolites 479M04, 479M08 and 479M12 were predicted to occur above the 0.75 µg/L threshold of concern and therefore require a refined risk assessment.

Metabolite 479M04

Acute toxicity and repeated dose toxicity studies conducted with 479M04 show that 479M04 is less toxic than parent metazachlor (see table B.6.8.2.1/1 Metazachlor Addendum – Confirmatory Data, 2013). The available dietary 28-day study in Wistar rats conducted with 479M04 identifies a NOAEL of 469 mg/kg bw/d. This NOAEL is 5-times higher than the NOAEL of 109 mg/kg bw/d identified for metazachlor in an equivalent dietary 28-day study in Wistar rats. Therefore, the use of the MAC (Maximum Allowable Concentration) value set for the parent (240 µg/L) for the risk assessment of 479M04 represents a conservative approach. Comparison of the MAC value of 240 µg/L with the highest PECgw of 4.700 µg/L predicted for 479M04 shows a 51-fold safety margin. On the basis of this margin, it is possible to conclude that metabolite 479M04 poses no unacceptable risks to consumers through drinking water.

Metabolite 479M08

Acute toxicity and repeated dose toxicity studies conducted with 479M08 show that 479M08 is less toxic than parent metazachlor (see table B.6.8.2.1/5 Metazachlor Addendum – Confirmatory Data, 2013). The available dietary 90-day study in Wistar rats conducted with 479M08 identifies a NOAEL of 282 mg/kg bw/d. This NOAEL is 17-times higher than the NOAEL of 16.7 mg/kg bw/d identified for metazachlor in an equivalent dietary 90-day study in Wistar rats. Therefore, the use of the MAC (Maximum Allowable Concentration) value set for the parent (240 µg/L) for the risk assessment of 479M08 represents a conservative approach. Comparison of the MAC value of 240 µg/L with the highest PECgw of 7.701 µg/L predicted for 479M08 shows a 31- fold safety margin. On the basis of this margin, it is possible to conclude that metabolite 479M08 poses no unacceptable risks to consumers through drinking water.

Metabolite 479M12

Acute toxicity and repeated dose toxicity studies conducted with 479M12 show that 479M12 is less toxic than parent metazachlor (see table B.6.8.2.1/4 Metazachlor Addendum – Confirmatory Data, 2013). The available dietary 28-day study in Wistar rats conducted with 479M12 identifies a NOAEL of 381 mg/kg bw/d. This NOAEL is ~4-times higher than the NOAEL of 109 mg/kg bw/d identified for metazachlor in an equivalent dietary 28-day study in Wistar rats. Therefore, the use of the MAC (Maximum Allowable Concentration) value set for the parent (240 µg/L) for the risk assessment of 479M12 represents a conservative approach. Comparison of the MAC value of 240 µg/L with the highest PECgw of 1.889 µg/L (after adjust) predicted for 479M12 shows a 127-fold safety margin. On the basis of this margin, it is possible to conclude that metabolite 479M12 poses no unacceptable risks to consumers through drinking water.

zRMS:

According to conclusions reached during the expert Meeting 151 (Peer review of the pesticide risk assessment for the active substance metazachlor in light of confirmatory data submitted (EFSA Journal 2017;15(6):4833) the metabolites 479M04(BH 479-4) and 479M08(BH 479-8) are not relevant up to stage 3 of step 3 of the guidance document (European Commission, 2003) since they do not share the early events associated with the carcinogenic effects observed with the parent, metazachlor. It had been previously agreed that the metabolite479M12 (BH 479-12) is not relevant.

The metabolites 479M09 (BH 479-9) and 479M11(BH 479-11) are relevant groundwater metabolites (EFSA, 2008a) should they occur in groundwater above the parametric drinking water limit of 0.1 g/L according to environmental exposure assessment. In case of application of a product SHA 148000 A / METROPOLITAN in accordance with GAP the PECgw values of these metabolites 479M09 and 479M11 are below 0.1 µg/L (also based on results of groundwater monitoring), thus they do not hamper registration of the product, since arising risk is considered acceptable because their concentrations in ground water is below 0.1 µg/L .

For three other  toxicologically not relevant metabolites 479M04, 479M08 and 479M12 in line with step 4 of the guidance document (European Commission), the following acceptable daily intake (ADI) values were established (EFSA Journal 2017;15(6):4833) to perform a step 5 of refined risk assessment for non-relevant metabolites:

- 0.33 mg/kg body weight (bw) per day for 479M04, based on a 90-day study in mice, applying an uncertainty factor (UF) of 1,000 to account for the limited data set;

- 0.2 mg/kg bw per day for 479M08, based on the developmental no observed adverse effect level (NOAEL) of 195 mg/kg bw per day from a developmental toxicity study in rats, applying the same UF of 1,000 to account for the limited data set; and

- 0.38 mg/kg bw per day for 479M12, based on the rat, 28-day and mouse, 90-day studies, UF 1,000 applied to account for the limited data set

According to Peer review of the pesticide risk assessment for the active substance metazachlor in light of confirmatory data submitted (EFSA Journal 2017;15(6):4833) for the use of the representative formulated products Butisan S (BAS 479 22H), and Fuego, both suspension concentrates (SC) containing 500 g/L metazachlor the predicted levels in ground water, the consumer exposure for the metabolites 479M04, 479M08 and 479M12 individually is around or less than 1% ADI for all considered consumer groups. Therefore, total intakes (from food and the drinking of groundwater) considering the chronic consumer exposure scenarios are expected to be well below the established toxicological reference values.

In case of application of the product SHA 148000 A / METROPOLITAN the following exposure and risk of consumers in drinking water might be expected assuming that consumer exposure assessment with regard to groundwater potentially used as drinking water is based on the default assumptions laid down in the WHO Guidelines (WHO, 2011) for drinking water quality for (a) a 60-kg adult drinking 2 L of water per day, (b) a 10-kg child drinking 1 L of water per day and (c) a 5-kg bottle-fed infant drinking 0.75 L of water per day:

Metabolite 479M04

Calculation of risk (% ADI) for 5-kg bottle-fed infant (consuming 0.75 l/day):

The predicted maximum level of 479M04 in groundwater is 4.7 µg/L.  Therefore, the potential intake of 479M04 by infants is 0.705 µg/kg bw (4.700 x 0.75 / 5).  This represents 0.21 % of the ADI for 479M04  (0.33 mg/kg bw/day).

Calculation of risk (% ADI) for 10-kg child (consuming 1.0 l/day):

The predicted maximum level of 479M04 in groundwater is 4.7 µg/L.  Therefore, the potential intake of 479M04 by children is 0.47 µg/kg bw (4.700 x 1/10).  This represents 0.14 % of the ADI for 479M04 (0.33 mg/kg bw/day).

Calculation of risk (% ADI) for 60-kg adult (consuming 2.0 l/day):

The predicted maximum level of 479M04 in groundwater is 4.7 µg/L.  Therefore, the potential intake of 479M04 by adults is 0.157 µg/kg bw (4.7 x 2 / 60).  This represents 0.47 % of the ADI for 479M04 (0.33 mg/kg bw/day).

Metabolite 479M08

Calculation of risk (% ADI) for 5-kg bottle-fed infant (consuming 0.75 l/day):

The predicted maximum level of 479M08 in groundwater is 7.701µg/L.  Therefore, the potential intake of 479M08 by infants is 1.155 µg/kg bw (7.701 x 0.75 / 5).  This represents 0.58 % of the ADI for 479M08  (0.2 mg/kg bw /day).

Calculation of risk (% ADI) for 10-kg child (consuming 1.0 l/day):

The predicted maximum level of 479M08 in groundwater is 7.701 µg/L.  Therefore, the potential intake of 479M08 by children is 0.77 µg/kg bw (7.701 x 1/10).  This represents 0.38 % of the ADI for 479M08 (0.2 mg/kg bw/day).

Calculation of risk (% ADI) for 60-kg adult (consuming 2.0 l/day):

The predicted maximum level of 479M08 in groundwater is 7.701µg/L.  Therefore, the potential intake of 479M08 by adults is 0.257 µg/kg bw (7.701 x 2 / 60).  This represents 0.13 % of the ADI for 479M08 (0.2 mg/kg bw/day).

Metabolite 479M12

Calculation of risk (% ADI) for 5-kg bottle-fed infant (consuming 0.75 l/day):

The predicted maximum level of 479M12 in groundwater is 1.889 µg/L.  Therefore, the potential intake of 479M12 by infants is 0.283 µg/kg bw (1.889 x 0.75/5).  This represents 0.074 % of the ADI for 479M12  (0.38 mg/kg bw /day).

Calculation of risk (% ADI) for 10-kg child (consuming 1.0 l/day):

The predicted maximum level of 479M12 in groundwater is 1.889 µg/L.  Therefore, the potential intake of 479M12 by children is 0.189 µg/kg bw (1.889 x 1/10).  This represents 0.049 % of the ADI for 479M12 (0.38 mg/kg bw/day).

Calculation of risk (% ADI) for 60-kg adult (consuming 2.0 l/day):

The predicted maximum level of 479M12 in groundwater is 1.889 µg/L.  Therefore, the potential intake of 479M12 by adults is 0.063 µg/kg bw (1.889 x 2 / 60).  This represents 0.016 % of the ADI for 479M12 (0.38 mg/kg bw/day).

Conclusion:

Noting that potential consumption of these metabolites is very low, well below 1% of relevant ADI set for each metabolites this  consumer risk assessment demonstrates an acceptable risk and a sufficient margin of safety to cover other potential exposure via other routes besides drinking water for these metabolites.

- Lists of data considered in support of the evaluation

- Additional information